Note: Work must be performed in accordance with this SWMS, and it must be available for inspection until the work to which this SWMS relates is completed. If the SWMS is revised, all versions are to be kept.

| TK Elevator Australia Pty Ltd – QLD ¦ ABN: 12 073 056 149 |  |  |  | SWMS # | M04 |
| --- | --- | --- | --- | --- | --- |
| Work Activity: | Access LMR via steep stairs or ladder |  |  | SWMS Issue Date: | <0/0/0> |
| Project: | <job name> |  |  | SWMS Review Date: | 20/08/25 |
| SWMS Developed By: | Peter Lawrence | Position: | QLD Safety Manager | Developed Date: | 25/09/20 |

| SWMS Approved By |  |  |  |
| --- | --- | --- | --- |
| Name: | Mazen Awyn | Signature: |  |
| Role / Position: | QLD NI/Mods Manager | Date: | <0/0/0> |

| High risk construction work: | Risk of a person falling more than 2m (note: in some jurisdictions this is 3m) |  | ☐ Work on a telecommunication tower |  |  |  | ☐ Work on a telecommunication tower |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ☐ Likely to involve disturbing asbestos |  | ☐ Temporary load-bearing support for structural alterations or repairs |  |  |  | ☐ Work in or near a confined space |  |
|  | ☐ Work in or near a shaft or trench deeper than 1.5m or a tunnel |  | ☐ Use of explosives |  |  |  | ☐ Work on or near pressurised gas mains or piping |  |
|  | ☐ Work on or near chemical, fuel or refrigerant lines |  | ☐ Work on or near energised electrical installations or services |  |  |  | ☐ Work in an area that may have a contaminated or flammable atmosphere |  |
|  | ☐ Tilt-up or precast concrete elements |  | ☐ Work on, in or adj to a road, railway, shipping lane or other traffic corridor in use by traffic other than pedestrians |  |  |  | ☐ Work in an area with movement of powered mobile plant |  |
|  | ☐ Work in areas with artificial extremes of temperature |  | ☐ Work in or near water or other liquid that involves a risk of drowning |  |  |  | ☐ Diving work |  |
| Change Management This SWMS must be reviewed when controls are potentially not effective (e.g. an incident occurs, work not in accordance with the SWMS etc.), changes to the workplace occur that introduce new hazards, or as required by the client’s system in order to keep the SWMS current to the work activity being undertaken. |  |  |  |  |  |  |  |  |  |
| Name |  | Signature |  | Date | Name | Signature |  | Date |  |

| Training/skills required |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inductions: | TKE |  | Client | ☐ |  | Site | ☐ | Other: |  |  |  | Vehicle/Plant | Licence | ☐ | Competency Assessment | ☐ | List: |
| Trade Qualification(s) | List: Electrical, Fitter |  |  |  | Task Specific Competencies |  |  |  |  | List: TKE GSM (Global Safety Manual) OHS Induction; Manual Handling Practices; General Construction Induction; Site Specific Induction (if applicable); Work Activity Induction – SWP (various); First Aid / CPR Training; Work at Heights Training |  |  |  |  |  |  |  |
| Equipment Required | List: lanyard, rope |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Health and Safety 10 Rules |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ☐ | ☐ | ☐ |  | ☐ | ☐ | ☐ |  | ☐ | ☐ |
| Lock Out / Tag Out (LOTO) | Fall Protection | Jumpers | PPE (specify below) | Car Top and Pit Access | Live Electrical / Troubleshooting | Mechanical Stored Energy | Rigging & Hoisting | False Cars & Running Platforms | Barricading |

| Personal Protective Equipment (PPE) – Required for task |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Uniform – long sleeves, long pants | Safety footwear AS/NZS 2210.3 | High-Vis AS/NZS 1906.4 | Hard hat AS/NZS 1801 | Safety eyewear AS/NZS 1337 | Gloves – to always be carried | Fall arrest | Hearing | Face mask (P2/N95) | Respirator (P2/N95) | Face shield | Low/High Voltage Gloves | Other (specify) |
|  |  | ☐ | ☐ |  |  | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ |

Risk Matrix

| Hierarchy of Control |  |  |
| --- | --- | --- |
| The hierarchy of control assists a person identify and choose a control that provides the highest level of safety possible. Note: there maybe more than one method of control. A control higher up the hierarchy will be a more effective control in the long run. Administrative and PPE controls are the least effective and only marginally manage the risk. |  |  |
| - Elimination – can the hazard be eliminated or completely removed? - If the hazard cannot be eliminated, then can the risk associated through the hazard be mitigated through one or a combination of one of the following? - Substitution – replacing a thing with something else. - Isolation – separating a person or object from something. - Modification / Re-Design – changing something or re-designing it in another way. Could be an object, process or something else. - Administration – includes work or written procedures, policies, rules, signage, training or information. - PPE (Personal Protective Equipment) – clothing or equipment designed to be worn to protect a person from the risk of injury or illness and can include helmets, ear plugs / muffs, eye or face protection, gloves, safety boots and respiratory protective equipment. | Risk Control Risk Management |  |

| Codes of Practice | Standards | Other |
| --- | --- | --- |
| - COP: Managing the Risk of Plant in the Workplace - COP: WH&S Consultation and Co-Operation - COP: Hazardous Manual Tasks - COP: How to Manage Health and Safety Risks - COP: Managing Noise Preventing Hearing Loss - COP: Managing the Work Environment and Facilities - COP: Preparation of Safety Data Sheet Hazardous Chemical - COP: Managing the risk of falls at workplaces - COP: Electrical Safety – Works - COP: Managing Electrical Risk in the Workplace - COP: Managing Respirable Crystalline Silica Dust in Construction and Manufacturing - COP: How to Control and Manage Asbestos in the Workplace - COP: Managing the risk of Psychosocial Hazards at Work - OH&S Induction Training for Construction Work - National Standard & Code of Practice for Manual Handling - National Standard & Code of Practice for Occupational Noise | - AS 1735.2:2001 – Passenger and Goods Lifts - Electric - AS 3760:2010 – In service safety inspection and testing of electrical equipment - AS 4431:2019 - Guidelines for safe working on new lift installations in new constructions - ISO31000 – Risk Management Principles - AS/NZS 3000:2018 – Electrical installations (Australian/New Zealand Wiring Rules) - AS/NZS 3012:2019 – Electrical installations - Construction and demolition sites - AS/NZS 3017:2007 – Electrical Installations – Verifications - AS/NZS 3760:2010 In Service Safety Inspection and Testing of Electrical Equipment - AS/NZS 4431:2019 Guidelines for Safe Working on New Lift Installations in New Constructions - AS/NZS 4836:2011 – Safe Working on or near Low-Voltage Electrical Installations and Equipment - UNI EN ISO 14798 – Lifts (elevators), escalators and moving walks - Risk assessment and reduction methodology. - I.S. EN 81-20:2020 Safety rules for the construction and installation of lifts - Lifts for the transport of persons and goods - Part 20: Passenger and goods passenger lifts - I.S. EN 81-50:2020 Safety rules for the construction and installation of lifts - Examinations and tests - Part 50: Design rules, calculations, examinations and tests of lift components | - Work Health and Safety Act 2011 (NSW, ACT & QLD) - Work Health and Safety Act 2012 (SA & TAS) - Electrical Safety Act 2002 - Electrical Safety Regulation 2013 - Occupational Health and Safety Act 2004 (WA & VIC) - Work Health and Safety Regulation (NSW) 2017 - Work Health & Safety Regulation (ACT) 2011 - Work Health & Safety Regulation (QLD) 2011 - Work Health & Safety Regulation (SA) 2012 - Work Health & Safety Regulation (TAS) 2012 - Occupational Health & Safety Regulations (WA & VIC) 2017 - TKE Aust – 10 Life Saving Rules |
|  |  | Mandatory SWP |
|  |  | - TKE SWP 1 – Use of Safety Equipment - TKE Aust – SWP No: 4 Materials Handling - TKE Aust – SWP No: 10 Working on Top of Lift Car - TKE Aust – SWP No: 15 Entering and Exiting Lift Pits - TKE Aust – SWP No: 16 Lift Car Top Access & Egress - TKE Aust – SWP No: 18 Lock Out Tag Out - TKE Aust – SWP No 29: Identification Guidelines and Management for ACM (Asbestos Containing Material) |

| Hazardous Substances to be brought on or kept on Site (SDS - Safety Data Sheet can be located on Branch SharePoint or VFM) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Product Name | SDS Present |  | Product Name | SDS Present |  |
|  | Yes☐ | No☐ |  | Yes☐ | No☐ |
|  | Yes☐ | No☐ |  | Yes☐ | No☐ |
|  | Yes☐ | No☐ |  | Yes☐ | No☐ |
|  | Yes☐ | No☐ |  | Yes☐ | No☐ |

| Relevant TKE AUS SWP for the task |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | SWP1 Use of Safety Equipment | ☐ | SWP2 General Electrical | ☐ | SWP3 Power Tools |
|  | SWP4 Materials Handling | ☐ | SWP5 Lifting Heavy Equipment | ☐ | SWP6 Hoisting & Rigging |
|  | SWP7 Working with Ladders | ☐ | SWP8 Working at Heights | ☐ | SWP9 Electrical Wiring |
| ☐ | SWP11 Working in Lift Motor Room |  | SWP10 Working on Top of Lift Car | ☐ | SWP12 Working in Lift Pit |
| ☐ | SWP13 Mechanical Fixing | ☐ | SWP14 Lift Shaft Entrance Barriers |  | SWP15 Entering & Exiting Lift Pits |
|  | SWP16 Lift Car Top Access and Egress |  | SWP17 Use of Height Safety Equipment |  | SWP18 Lock Out and Tag Out |
| ☐ | SWP19 Scaffold Erection | ☐ | SWP20 Working on Scaffold | ☐ | SWP21 Winch Operation |
| ☐ | SWP22 Welding & Oxy Cutting | ☐ | SWP23 Working with Escalators & Moving Walks | ☐ | SWP24 Dismantling & Removing Equipment |
| ☐ | SWP25 Replacing Ropes | ☐ |  | ☐ | SWP27 Other Contractors in Lift Shaft or Motor Room |
| ☐ | SWP28 Other Contractors in Lift Car |  | SWP29 Asbestos, Mineral Fibres, Hazardous Dust | ☐ | SWP30 Trapped Passenger Rescue – Manual Doors |
| ☐ | SWP31 Trapped Passenger Rescue – Hydraulic Lifts | ☐ | SWP32 Trapped Passenger Rescue – Geared Traction | ☐ | SWP33 Removing Lift Machine (MRL) |
| ☐ | SWP 34 Installing Ropes | ☐ | SWP35 Fitting of Rails | ☐ | SWP 36 Fitting of Door Frames |
| ☐ | SWP 37 Assembly of Car & Counterweight Frames | ☐ | SWP 38 Use of False Cars | ☐ | SWP 39 Re-roping |
| ☐ | SWP 39 Re-roping | ☐ | SWP40 Replace Top of Car Sheave |  | SWP41 Working in Corridors and Access Ways |
| ☐ | SWP42 Using Electric Power Tools | ☐ | SWP43 Weighing Lift Cars | ☐ | SWP44 MRL Machine Maintenance |
| ☐ | SWP45 Working Alone | ☐ | SWP46 Safe Working on Electrical Equipment | ☐ | SWP47 Stopped with Passengers Callouts |
| ☐ | SWP48 Resetting Tail Rope Safety Gear | ☐ | SWP49 Testing Safety Gear | ☐ | SWP50 Pit Access and Egress – Lift Not in Service |
| ☐ | SWP51 Drive Otis 401 Lift from Motor Room – No Inspection Buttons Fitted | ☐ | SWP52 Fall Arrest Rescue | ☐ | SWP53 Using Door Wedges |
| ☐ | SWP54 SWIFT Bypass Button Operation | ☐ | SWP55 Permanent Magnet Filler Weightlifter Operation | ☐ | SWP56 Working on Dumbwaiters |
| ☐ | SWP57 Slow Speed Testing of Safety Gear | ☐ | SWP58 Use of Bridging Leads | ☐ | SWP59 Use of RB Anchor |
| ☐ | SWP60 Erection of Temporary Scaffold | ☐ | SWP61 Testing Evolution Safety Gear Governor | ☐ | SWP62 Public Safety on Escalators |
| ☐ | SWP63 Warehouse Safety | ☐ | SWP64 Dual Plunger Brake Test for Service | ☐ | SWP65 Taking Command of a Lift for Test and Maintenance |
| ☐ | SWP66 Motor Room Access via Caged Ladder | ☐ | SWP67 Gas Operated Nail Gun | ☐ | SWP68 Use of Hazardous Substances |
|  | SWP69 Motor Room Access via Ladder | ☐ | SWP70 How to lift Escalator Hatch using TKEA Hook & Wedge | ☐ | SWP71 Use of False Cars in Open Atrium Shaft |
| ☐ |  | ☐ | SWP73 Trapped Passenger Rescue - MRL |  | SWP74 Accessing Buildings |
| ☐ | SWP75 Spill Control of Hazardous Substances | ☐ | SWP76 Lift Shaft Access EVO GEN 1 Drawbridge |  | SWP77 Access via Ladder all areas |
| ☐ | SWP78 Working in Open Atrium Top of the Lift Car | ☐ | SWP79 Manual Operation of Holister Whitney Rope Gripper | ☐ | SWP80 Working on top of a Lift Car - TWIN |
| ☐ | SWP81 Replacing Hydraulic Ram | ☐ | SWP82 Installation of Pit Prop | ☐ | SWP83 Installation of Rope Savers |
| ☐ | SWP84 Removal of Injured Person from False Car | ☐ | SWP85 Removal of Recycle material from TKEA Store | ☐ | SWP86 Review of Subcontractor SWMS |
| ☐ | SWP87 Low Voltage Rescue | ☐ | SWP88 PCB’s | ☐ | SWP89 Raising Electric Winch into Position |
| ☐ | SWP90 Cleaning a Lift Motor Room | ☐ | SWP91 Installation of an Elevator Car | ☐ | SWP92 Installation of Temporary Hoardings |
| ☐ | SWP93 Rail Clamps | ☐ | SWP94 Use of Hoardings on MOD Sites | ☐ | SWP95 Door Locks Out of Circuit |
| ☐ |  | ☐ |  | ☐ |  |

| Job Step Break the job down into logical steps |  | Potential Hazards Identify the hazards associated with each step. Examine each to find possibilities that could lead to an accident. | Initial Risk Level | Controls List the actions necessary to eliminate or minimise the hazards that could lead to an accident, injury, or occupational illness. | Final Risk Level | Person Responsible Identify the Persons responsible for ensuring controls are implemented. |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Notify client Move materials and tools to the top floor Conduct risk assessment and plan movement of materials. | Client staff and public | 40 | Upon arrival, the TKE supervisor shall contact the nominated facility representative. | 6 | TKE Personnel TKE Supervisor |
|  |  |  |  | All TKE staff must attend relevant site inductions before commencing work on-site. |  |  |
|  |  |  |  | Access to and from the facilities, including accessways, will not be compromised during the works. |  |  |
|  |  |  |  | Barricading and delineation will separate the public and staff from the work area during each phase of the work. |  |  |
|  |  |  |  | Pre-start meetings and toolbox meetings are to be completed. |  |  |
|  |  |  |  | No workers will attend work if affected by alcohol or drugs. |  |  |
|  |  |  |  | TKE staff are to follow all reasonable instructions from the facility management/representative. |  |  |
|  |  |  |  | TKE staff are to follow all reasonable instructions from the facility management/representative. |  |  |
|  |  |  |  | During work, TKE personnel will implement all appropriate permit systems the facility uses. These may include hot works, electrical isolation, and works involving the disturbance of hazardous materials. |  |  |
|  |  |  |  | TKE personnel will lease with the facility management to ensure that all required service isolations are in place before each phase of work, including but not limited to - Mains power - Water - Fire systems. |  |  |
|  |  | Manual Handling |  | Use appropriate PPE. |  |  |
|  |  |  |  | Always use correct lifting techniques. |  |  |
|  |  |  |  | Where possible, use mechanical aids for lifting. |  |  |
|  |  |  |  | Refer to SWP 4 |  |  |
| 2 | Access the Lift motor room via steep stairs or a ladder. | Slips Trips | 40 | Refer to SWP 7 | 6 | TKE Personnel |
|  |  |  |  | Refer to SWP 41 |  |  |
|  |  |  |  | Refer to SWP 69 |  |  |
|  |  |  |  | Refer to SWP 77 |  |  |
|  |  | Falls |  | Always maintain 3 points of contact while using a ladder. |  |  |
|  |  |  |  | Inspect ladder rungs or stair treads for grease/mud. |  |  |
|  |  |  |  | Ensure handrails are in place and in good condition on the stairs. |  |  |
| 3 | Move tools to lift the motor room | Manual Handling | 50 | Use appropriate PPE. | 10 | TKE Personnel |
|  |  |  |  | Always use correct lifting techniques. |  |  |
|  |  |  |  | Where possible, use mechanical aids for lifting. |  |  |
|  |  |  |  | Refer to SWP 4 |  |  |
|  |  | Objects fall from heights. |  | Materials under 10kg should be lifted using a lowering rope. With a second technician on the lower level and 1-2 technicians lifting the items |  |  |
|  |  |  |  | Break oversized items into smaller components. |  |  |
|  |  |  |  | Heavier items should be raised using hatchways and winch/chain blocks. |  |  |
|  |  |  |  | All lifting gear to be tagged and checked before use |  |  |
|  |  |  |  | All rigging work is to be performed by a licenced rigger. |  |  |
| 4 | Remove tools and equipment. | Revers process of steps 2 and 3 |  |  |  |  |
| 5 | Leave the lift motor room. General clean up, remove tools and waste | Cuts, Abrasions | 14 | Use cut-resistant gloves. | 6 | TKE Personnel |
|  |  | Hazardous substances |  | Use correct PPE. |  |  |
|  |  |  |  | Ensure a spill kit is at hand. |  |  |
|  |  |  |  | Follow all controls outlined in SDS. |  |  |
|  |  | Manual Tasks |  | Use correct Lifting Techniques. |  |  |
|  |  |  |  | Use mechanical lifting devices where possible. |  |  |
|  |  |  |  | Use correct manual handling techniques. |  |  |
|  |  |  |  | Where possible, Use the two-man lift principle. |  |  |

| SWMS Sign Off I have been consulted and given the opportunity to contribute to this SWMS. I confirm that I understood the contents of this SWSM. I have read and understand the relevant SWP related to this SWMS. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | Signature | Date | Name | Signature | Date |
|  |  | / / |  |  | / / |
|  |  | / / |  |  | / / |
|  |  | / / |  |  | / / |
|  |  | / / |  |  | / / |
|  |  | / / |  |  | / / |
|  |  | / / |  |  | / / |
|  |  | / / |  |  | / / |
|  |  | / / |  |  | / / |
|  |  | / / |  |  | / / |
|  |  | / / |  |  | / / |
|  |  | / / |  |  | / / |
|  |  | / / |  |  | / / |
|  |  | / / |  |  | / / |
|  |  | / / |  |  | / / |