Note: Work must be performed in accordance with this SWMS, and it must be available for inspection until the work to which this SWMS relates is completed. If the SWMS is revised, all versions are to be kept.

| TK Elevator Australia Pty Ltd – QLD ¦ ABN: 12 073 056 149 |  |  |  |  |  |  |  | SWMS # | NI-M100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Work Activity: | Meta 100 – Installation of META 100 Lift |  |  |  |  |  |  | SWMS Issue Date: | <0/0/0> |
| Project: | <job name> |  |  |  |  |  |  | SWMS Review Date: | 12/06/24 |
| SWMS Developed By: | Peter Lawrence |  |  | Position: |  | QLD Safety Manager |  | Developed Date: | 28/04/22 |
| Workers Consulted (employees, SME) |  | Position | Date |  | SWMS Review Team |  |  | Position | Date |
| Izaac Byrne |  | Technician | 20/3/24 |  | Micheal Havilah |  | QLD NI/Mods Supervisor |  | 22/03/24 |
| Micheal Havilah |  | Technician | 20/3/24 |  | Frank Arfuso |  | QLD NI/Mods Supervisor |  | 22/03/24 |
| Brad Hanson |  | Technician | 20/3/24 |  | Mazen Awyn |  | QLD NI/Mods Manager |  | 22/03/24 |

| SWMS Approved By |  |  |  |
| --- | --- | --- | --- |
| Name: | Mazen Awyn | Signature: |  |
| Role / Position: | QLD NI/Mods Manager | Date: | <0/0/0> |

| High risk construction work: | Risk of a person falling more than 2m (note: in some jurisdictions this is 3m) | ☐ Work on a telecommunication tower | ☐ Work on a telecommunication tower |
| --- | --- | --- | --- |
|  | ☐ Likely to involve disturbing asbestos | ☐ Temporary load-bearing support for structural alterations or repairs | ☐ Work in or near a confined space |
|  | Work in or near a shaft or trench deeper than 1.5m or a tunnel | ☐ Use of explosives | ☐ Work on or near pressurised gas mains or piping |
|  | ☐ Work on or near chemical, fuel or refrigerant lines | ☐ Work on or near energised electrical installations or services | ☐ Work in an area that may have a contaminated or flammable atmosphere |
|  | ☐ Tilt-up or precast concrete elements | ☐ Work on, in or adj to a road, railway, shipping lane or other traffic corridor in use by traffic other than pedestrians | ☐ Work in an area with movement of powered mobile plant |
|  | ☐ Work in areas with artificial extremes of temperature | ☐ Work in or near water or other liquid that involves a risk of drowning | ☐ Diving work |

| Change Management This SWMS must be reviewed when controls are potentially not effective (e.g. an incident occurs, work not in accordance with the SWMS etc.), changes to the workplace occur that introduce new hazards, or as required by the client’s system to keep the SWMS current to the work activity being undertaken. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | Signature | Date | Name | Signature | Date |

| Training/skills required |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inductions: | TKE |  | Client | ☐ | Site |  | ☐ | Other: |  |  | Vehicle/Plant |  | Licence | ☐ | Competency Assessment | ☐ | List: |
| Trade Qualification(s) | List: Electrical license, Cert III Engineering (mechanical trade) |  |  |  |  | Task Specific Competencies |  |  |  |  |  | List: TKE GSM (Global Safety Manual) OHS Induction; Manual Handling Practices; General Construction Induction; Site Specific Induction (if applicable); Work Activity Induction – SWP (various); First Aid / CPR Training; Work at Heights Training, Basic Rigger |  |  |  |  |  |
| Equipment Required | List: forklift, pallet jack, lifting aids, Multimeter, Hand Tools, Rigging gear |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Health and Safety 10 Rules |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ☐ |  |  |  |  |
| Lock Out / Tag Out (LOTO) | Fall Protection | Jumpers | PPE (specify below) | Car Top and Pit Access | Live Electrical / Troubleshooting | Mechanical Stored Energy | Rigging & Hoisting | False Cars & Running Platforms | Barricading |

| Personal Protective Equipment (PPE) – Required for task |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Uniform – long sleeves, long pants | Safety footwear AS/NZS 2210.3 | High-Vis AS/NZS 1906.4 | Hard hat AS/NZS 1801 | Safety eyewear AS/NZS 1337 | Gloves – to always be carried | Fall arrest | Hearing | Face mask (P2/N95) | Respirator (P2/N95) | Face shield | Low/High Voltage Gloves | Other (specify) |
|  |  | ☐ |  |  |  | ☐ |  | ☐ | ☐ | ☐ | ☐ | ☐ |

| Risk Matrix |  |  |
| --- | --- | --- |
| Hierarchy of Control |  |  |
| The hierarchy of control assists a person identify and choose a control that provides the highest level of safety possible. Note: there maybe more than one method of control. A control higher up the hierarchy will be a more effective control in the long run. Administrative and PPE controls are the least effective and only marginally manage the risk. |  |  |
| - Elimination – can the hazard be eliminated or completely removed? - If the hazard cannot be eliminated, then can the risk associated through the hazard be mitigated through one or a combination of one of the following? - Substitution – replacing a thing with something else. - Isolation – separating a person or object from something. - Modification / Re-Design – changing something or re-designing it in another way. Could be an object, process or something else. - Administration – includes work or written procedures, policies, rules, signage, training or information. - PPE (Personal Protective Equipment) – clothing or equipment designed to be worn to protect a person from the risk of injury or illness and can include helmets, ear plugs / muffs, eye or face protection, gloves, safety boots and respiratory protective equipment. | Risk Control Risk Management |  |

| Codes of Practice | Standards | Other |
| --- | --- | --- |
| - COP: Managing the Risk of Plant in the Workplace - COP: WH&S Consultation and Co-Operation - COP: Hazardous Manual Tasks - COP: How to Manage Health and Safety Risks - COP: Managing Noise Preventing Hearing Loss - COP: Managing the Work Environment and Facilities - COP: Preparation of Safety Data Sheet Hazardous Chemical - COP: Managing the risk of falls at workplaces - COP: Electrical Safety – Works - COP: Managing Electrical Risk in the Workplace - COP: Managing Respirable Crystalline Silica Dust in Construction and Manufacturing - COP: How to Control and Manage Asbestos in the Workplace - COP: Managing the risk of Psychosocial Hazards at Work - OH&S Induction Training for Construction Work - National Standard & Code of Practice for Manual Handling - National Standard & Code of Practice for Occupational Noise | - AS 1735.2:2001 – Passenger and Goods Lifts - Electric - AS 3760:2010 – In service safety inspection and testing of electrical equipment - AS 4431:2019 - Guidelines for safe working on new lift installations in new constructions - ISO31000 – Risk Management Principles - AS/NZS 3000:2018 – Electrical installations (Australian/New Zealand Wiring Rules) - AS/NZS 3012:2019 – Electrical installations - Construction and demolition sites - AS/NZS 3017:2007 – Electrical Installations – Verifications - AS/NZS 3760:2010 In Service Safety Inspection and Testing of Electrical Equipment - AS/NZS 4431:2019 Guidelines for Safe Working on New Lift Installations in New Constructions - AS/NZS 4836:2011 – Safe Working on or near Low-Voltage Electrical Installations and Equipment - UNI EN ISO 14798 – Lifts (elevators), escalators and moving walks - Risk assessment and reduction methodology. - I.S. EN 81-20:2020 Safety rules for the construction and installation of lifts - Lifts for the transport of persons and goods - Part 20: Passenger and goods passenger lifts - I.S. EN 81-50:2020 Safety rules for the construction and installation of lifts - Examinations and tests - Part 50: Design rules, calculations, examinations and tests of lift components | - Work Health and Safety Act 2011 (NSW, ACT & QLD) - Work Health and Safety Act 2012 (SA & TAS) - Electrical Safety Act 2002 - Electrical Safety Regulation 2013 - Occupational Health and Safety Act 2004 (WA & VIC) - Work Health and Safety Regulation (NSW) 2017 - Work Health & Safety Regulation (ACT) 2011 - Work Health & Safety Regulation (QLD) 2011 - Work Health & Safety Regulation (SA) 2012 - Work Health & Safety Regulation (TAS) 2012 - Occupational Health & Safety Regulations (WA & VIC) 2017 - TKE Aust – 10 Life Saving Rules |
|  |  | Mandatory SWP |
|  |  | - TKE SWP 1 – Use of Safety Equipment - TKE Aust – SWP No: 4 Materials Handling - TKE Aust – SWP No: 10 Working on Top of Lift Car - TKE Aust – SWP No: 15 Entering and Exiting Lift Pits - TKE Aust – SWP No: 16 Lift Car Top Access & Egress - TKE Aust – SWP No: 18 Lock Out Tag Out - TKE Aust – SWP No 29: Identification Guidelines and Management for ACM (Asbestos Containing Material) |

| Hazardous Substances to be brought on or kept on Site (SDS - Safety Data Sheet can be located on Branch SharePoint, VFM or ChemAlert) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Product Name | SDS Present |  | Product Name | SDS Present |  |
| SHELLITE | Yes | No☐ |  | Yes☐ | No☐ |
| DIGGERS MINERAL TURPENTINE | Yes | No☐ |  | Yes☐ | No☐ |
| WHITE KNIGHT ULTRA PAVE QUICK DRY MEDIUM GREY | Yes | No☐ |  | Yes☐ | No☐ |
| HYDRAULIC 68 | Yes | No☐ |  | Yes☐ | No☐ |

| Relevant TKE AUS SWP for the task |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | SWP1 Use of Safety Equipment | ☐ | SWP2 General Electrical | ☐ | SWP3 Power Tools |
|  | SWP4 Materials Handling | ☐ | SWP5 Lifting Heavy Equipment | ☐ | SWP6 Hoisting & Rigging |
| ☐ | SWP7 Working with Ladders | ☐ | SWP8 Working at Heights | ☐ | SWP9 Electrical Wiring |
| ☐ | SWP11 Working in Lift Motor Room |  | SWP10 Working on Top of Lift Car | ☐ | SWP12 Working in Lift Pit |
| ☐ | SWP13 Mechanical Fixing | ☐ | SWP14 Lift Shaft Entrance Barriers |  | SWP15 Entering & Exiting Lift Pits |
|  | SWP16 Lift Car Top Access and Egress | ☐ | SWP17 Use of Height Safety Equipment |  | SWP18 Lock Out and Tag Out |
| ☐ | SWP19 Scaffold Erection | ☐ | SWP20 Working on Scaffold | ☐ | SWP21 Winch Operation |
| ☐ | SWP22 Welding & Oxy Cutting | ☐ | SWP23 Working with Escalators & Moving Walks | ☐ | SWP24 Dismantling & Removing Equipment |
| ☐ | SWP25 Replacing Ropes | ☐ |  | ☐ | SWP27 Other Contractors in Lift Shaft or Motor Room |
| ☐ | SWP28 Other Contractors in Lift Car |  | SWP29 Asbestos, Mineral Fibres, Hazardous Dust | ☐ | SWP30 Trapped Passenger Rescue – Manual Doors |
| ☐ | SWP31 Trapped Passenger Rescue – Hydraulic Lifts | ☐ | SWP32 Trapped Passenger Rescue – Geared Traction | ☐ | SWP33 Removing Lift Machine (MRL) |
| ☐ | SWP 34 Installing Ropes | ☐ | SWP35 Fitting of Rails | ☐ | SWP 36 Fitting of Door Frames |
| ☐ | SWP 37 Assembly of Car & Counterweight Frames | ☐ | SWP 38 Use of False Cars | ☐ | SWP 39 Re-roping |
| ☐ | SWP 39 Re-roping | ☐ | SWP40 Replace Top of Car Sheave | ☐ | SWP41 Working in Corridors and Access Ways |
| ☐ | SWP42 Using Electric Power Tools | ☐ | SWP43 Weighing Lift Cars | ☐ | SWP44 MRL Machine Maintenance |
| ☐ | SWP45 Working Alone | ☐ | SWP46 Safe Working on Electrical Equipment | ☐ | SWP47 Stopped with Passengers Callouts |
| ☐ | SWP48 Resetting Tail Rope Safety Gear | ☐ | SWP49 Testing Safety Gear | ☐ | SWP50 Pit Access and Egress – Lift Not in Service |
| ☐ | SWP51 Drive Otis 401 Lift from Motor Room – No Inspection Buttons Fitted | ☐ | SWP52 Fall Arrest Rescue | ☐ | SWP53 Using Door Wedges |
| ☐ | SWP54 SWIFT Bypass Button Operation | ☐ | SWP55 Permanent Magnet Filler Weightlifter Operation | ☐ | SWP56 Working on Dumbwaiters |
| ☐ | SWP57 Slow Speed Testing of Safety Gear | ☐ | SWP58 Use of Bridging Leads | ☐ | SWP59 Use of RB Anchor |
| ☐ | SWP60 Erection of Temporary Scaffold | ☐ | SWP61 Testing Evolution Safety Gear Governor | ☐ | SWP62 Public Safety on Escalators |
| ☐ | SWP63 Warehouse Safety | ☐ | SWP64 Dual Plunger Brake Test for Service | ☐ | SWP65 Taking Command of a Lift for Test and Maintenance |
| ☐ | SWP66 Motor Room Access via Caged Ladder | ☐ | SWP67 Gas Operated Nail Gun |  | SWP68 Use of Hazardous Substances |
| ☐ | SWP69 Motor Room Access via Ladder | ☐ | SWP70 How to lift Escalator Hatch using TKEA Hook & Wedge | ☐ | SWP71 Use of False Cars in Open Atrium Shaft |
| ☐ |  | ☐ | SWP73 Trapped Passenger Rescue - MRL | ☐ | SWP74 Accessing Buildings |
| ☐ | SWP75 Spill Control of Hazardous Substances | ☐ | SWP76 Lift Shaft Access EVO GEN 1 Drawbridge | ☐ | SWP77 Access via Ladder in all areas |
| ☐ | SWP78 Working in Open Atrium Top of the Lift Car | ☐ | SWP79 Manual Operation of Holister Whitney Rope Gripper | ☐ | SWP80 Working on top of a Lift Car - TWIN |
| ☐ | SWP81 Replacing Hydraulic Ram | ☐ | SWP82 Installation of Pit Prop | ☐ | SWP83 Installation of Rope Savers |
| ☐ | SWP84 Removal of Injured Person from False Car | ☐ | SWP85 Removal of Recycle material from TKEA Store | ☐ | SWP86 Review of Subcontractor SWMS |
| ☐ | SWP87 Low Voltage Rescue | ☐ | SWP88 PCB’s | ☐ | SWP89 Raising Electric Winch into Position |
| ☐ | SWP90 Cleaning a Lift Motor Room | ☐ | SWP91 Installation of an Elevator Car |  | SWP92 Installation of Temporary Hoardings |
| ☐ | SWP93 Rail Clamps |  | SWP94 Use of Hoardings on MOD Sites | ☐ | SWP95 Door Locks Out of Circuit |
| ☐ |  | ☐ |  | ☐ |  |

Contents

Activity 1 - Site Establishment	8

Activity 2 - Installation Preparation / Site Set Up	10

Activity 3 - Concrete Hook Test	12

Activity 4 - Template Installation (if using lasers, plumb lines setting with plumb lasers method is applicable to the shaft heights no more than 30 meters)	28

Activity 5 - Pit Components Installation	37

Activity 6 - Counterweight Frame Installation	46

Activity 7 - Car Sling and Cabin Installation	51

Activity 8 - Car Top Hoisting Assembly Installation	61

Activity 9 - Blocstop-Safety Gear Activation Test	71

Activity 10 - Other Special Toolings Installation (Guide Devices and Fixing Fixture Installation and Rail Transport Toolings Installation)	74

Activity 11 - Remaining Guide Rails Installation (Guide Rails and Counterweight Rails)	83

Activity 12 Landing Doors Installation	96

Activity 13 - Overhead Equipment Installation	98

Activity 14 - Lift Shaft Electrical Components Installation	108

Activity 15 - Traction Machine Roping	119

Activity 16 - Counterweight Fillers Top-Up	136

Activity 17 - Overspeed Governor Roping	143

Activity 18 - Inspection Running Commissioning	150

Activity 19 - Hoisting Assembly Dismantlement	154

Activity 20 - Other Elevator Components Installation (Install Car Electricals)	158

Activity 21 - Normal Speed Running Commissioning	164

SWMS Sign Off	179

| # Activity 1 - Site Establishment |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Job Step Break the job down into logical steps |  | Potential Hazards Identify the hazards associated with each step. Examine each to find possibilities that could lead to an accident. | Initial Risk Level | Controls List the actions necessary to eliminate or minimise the hazards that could lead to an accident, injury, or occupational illness. | Final Risk Level | Person Responsible Identify the Persons responsible for ensuring controls are implemented. |
| 1 | Site Arrival / General | Fire or other emergency requiring evacuation of the site or other e.g., Lift out of Service or SWP (Stop with Passenger) | 40 | - TKE personnel inducted to site emergency and evacuation procedures e.g. aware of alarm system and evacuation assembly locations. | 10 | TKE Personnel |
|  |  |  |  | - TKE personnel to notify Site Security if the lift is out of service or trapped passengers. |  |  |
|  |  |  |  | - TKE personnel to abide by Site Conditions of Entry and Traffic Management Plan. |  |  |
|  |  |  |  | - Site contact notified upon arrival |  |  |
|  |  | General risk of injury to Public and Staff from work involving Lift servicing – uncontrolled access to lift environment e.g. control panel, lift shaft etc. | 20 | - Access to the Control Panel and Lift Shaft is locked and secured when lift personnel are not in the vicinity. | 5 | TKE Personnel |
|  |  |  |  | - Areas of work isolated or barricaded from the public or staff if in the public access area. |  |  |
|  |  | Risk of Injury to TK Elevator personnel. | 50 | - Pre-Start Risk Assessment to be completed before commencement of work. | 10 | TKE TKE Personnel Building Manager |
|  |  |  |  | - TKE personnel trained as First Aid Attendants and equipped with portable first aid kits and mobile phones. |  |  |
|  |  |  |  | - PPE (safety glasses and/or goggles) to be worn always when working in the Shaft, Top of the car, and Pit |  |  |
|  |  |  |  | - Dependent on task – some Tasks require the presence of at least 2 personnel. |  |  |
|  |  |  |  | - Injuries & incidents were immediately reported to the Building Manager, Supervisor, and TKE Safety Team. Refer to TKE S147(a) Accident and Incident Report. |  |  |
|  |  |  |  | - If an incident/accident is defined as serious and notifiable under QLD WHS legislation, TKE to notify SafeWork QLD in consultation with the Building Manager. |  |  |
|  |  | Risk of injury or incident involving TK personnel or Contractor to 3rd party (member of public or site employee) through mechanical malfunction or failure | 40 | - Routine Repairs and Servicing conducted only by trained and licensed lift personnel. | 10 | TKE Personnel |
|  |  |  |  | - Maintenance and serving per Manufacturer’s instructions or Industry best practices and standards. |  |  |
|  |  | Risk of exposure to Hazardous Substances (Asbestos or Asbestos Containing Material – ACM) through servicing or repair activities. ACM is often located in switches arc shields etc. | 30 | - TKE Personnel trained in Asbestos Awareness. | 6 | TKE Management TKE Personnel |
|  |  |  |  | - Copy of Site Asbestos Register available and reviewed to determine if ACM (asbestos-containing material) is present in the Lift environment. May be present in the environment and componentry e.g., limit switches and brakes. |  |  |
|  |  |  |  | - If suspicious material is identified and confirmation of asbestos cannot be verified from the register or labelling, then work is to cease, and the material is to be tested and confirmed as asbestos by a licensed laboratory before work continues (per TKE SWP 29 and 68). |  |  |

| # Activity 2 - Installation Preparation / Site Set Up |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Job Step Break the job down into logical steps |  | Potential Hazards Identify the hazards associated with each step. Examine each to find possibilities that could lead to an accident. | Initial Risk Level | Controls List the actions necessary to eliminate or minimise the hazards that could lead to an accident, injury, or occupational illness. | Final Risk Level | Person Responsible Identify the Persons responsible for ensuring controls are implemented. |
| 1 | Conduct a preliminary inspection of the site | Risk of slip and trip | 40 | - Remove any trip hazards present. | 10 | TKE Personnel |
|  |  |  |  | - Ensure adequate lighting in all access areas. |  |  |
| 2 | Prepare, distribute, hoist, and transport materials and equipment to required areas | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Vehicles with Cranes or Tailgate lifters to be used to transport equipment to the site. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of members of the public or Tenants being injured | 28 | - Ensure a clear path from truck to site | 14 | TKE Personnel |
|  |  |  |  | - Spotter to be used |  |  |
|  |  |  |  | - Barricading to be set around a delivery truck |  |  |
|  |  | Risk of being struck by a car | 40 | - Spotter to be used | 10 | TKE Personnel |
|  |  |  |  | - Park in the loading zone or allocated zone |  |  |
| 3 | Hooks and anchor points are to be made available and certified per regulatory guidelines | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |
| 4 | Install shaft entrance protection (such as hoarding) at each landing firmly fixed. Signage is to be posted on each landing. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of injury to Third Party / Building Tenants through unsecured access or unstable fencing/hoarding. | 20 | - Install hoarding per supplier’s instructions and as per SWP 92 and 94. Ensure hoarding is secured at the base and top to door header or another fixture. | 5 | TKE Personnel |
|  |  |  |  | - Ensure the hoarding access door is lockable and secure. |  |  |
|  |  |  |  | - Inspect the area where the hoarding is to be installed. Isolate and barricade the work area from the public and staff. Open doors to Lift Shaft not to be left unattended. |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 10 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |

| # Activity 3 - Concrete Hook Test |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Job Step Break the job down into logical steps |  | Potential Hazards Identify the hazards associated with each step. Examine each to find possibilities that could lead to an accident. | Initial Risk Level | Controls List the actions necessary to eliminate or minimise the hazards that could lead to an accident, injury, or occupational illness. | Final Risk Level | Person Responsible Identify the Persons responsible for ensuring controls are implemented. |
| 1 | Install top-floor barriers before commencing this activity. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of fall from height from landing into shaft/pit | 50 | - Fall protection to be worn when installing a barrier. | 10 | TKE Personnel |
|  |  |  |  | - Fall protection to be inspected every 6 months and to be in good condition. |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 10 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |
| 2 | Assemble lifting rods by connecting the two ends with safety pins. Cont. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
| 3 | Connect blocstop ropes with the bracket of the guide pulley (pulling out a sufficient length of the blocstop rope) Connect the lifting rod to the hook of the blocstop rope (fastening in a clockwise direction) | Risk of fall from height from landing into shaft/pit | 50 | - Barrier fixed in place at the entrance to shaft. - Barrier to be checked before commencing work | 10 | TKE Personnel |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
| 4 | Hang the hook of the blocstop rope onto the concrete hook with the lifting rod (ensuring the blocstop hook is reliably hung onto the concrete hook of the overspeed governor; the self-locking mechanism is engaged when the hook is in place and the guide pulley stays on the landing floor) | Risk of fall from height from landing into shaft/pit | 50 | - Barrier fixed in place at the entrance to shaft. | 10 | TKE Personnel |
|  |  |  |  | - Barrier to be checked before commencing work |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment. |  |  |
| 5 | Remove the lifting rod and lower the blocstop rope into the pit (roll back and keep excess rope in the rope reel which is fastened in the pit or outside the shaft) | Risk of fall from height from landing into shaft/pit | 50 | - Barrier fixed in place at the entrance to shaft. | 10 | TKE Personnel |
|  |  |  |  | - Barrier to be checked before commencing work. |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
| 6 | For the PSDW Steel Wire Rope Installation, pull out the PSDW rope and thread it through the groove of the guide pulley. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
| 7 | Connect the lifting rod to the hook of the guide pulley and hang the guide pulley onto the concrete hook. | Risk of fall from height from landing into shaft/pit | 50 | - Barrier fixed in place at the entrance to shaft. | 10 | TKE Personnel |
|  |  |  |  | - Barrier to be checked before commencing work |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment. |  |  |
| 8 | Remove the lifting rod and ensure the layout of equipment at the top of the shaft is as below: | Risk of fall from height from landing into shaft/pit | 50 | - Barrier fixed in place at the entrance to shaft. | 10 | TKE Personnel |
|  |  |  |  | - Barrier to be checked before commencing work |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment. |  |  |
| 9 | Lower the PSDW rope into the pit and completely pull out the PSDW rope of the other end from the rope reel and lower it to the pit as well. | Risk of fall from height from landing into shaft/pit | 50 | - Barrier fixed in place at the entrance to shaft. | 10 | TKE Personnel |
|  |  |  |  | - Barrier to be checked before commencing work. |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
| 10 | Access pi | Risk of medical or another emergency involving a Technician descending the ladder or while working in the pit. | 40 | - Alternative 2-way communication used if visual not available e.g. Technician on Top of Car. | 10 | TKE Personnel |
|  |  |  |  | - “Man Down” feature on Smartphone to be used to alert and notify of medical emergencies. |  |  |
|  |  |  |  | - Emergency Services are to be contacted and the Building Manager is advised in the event of a medical emergency and/or if evacuation from the pit is required. |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Before descending the ladder, conduct a visual check of the condition of the floor and ladder and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
|  |  | Risk of injury from Falling Objects – objects falling from landing onto the Technician in the Pit. | 28 | - No items stored in the vicinity of the edge when doors open. | 14 | TKE Personnel |
|  |  |  |  | - Landing doors closed to a maximum width of 200mm once Technician One has entered the pit. |  |  |
|  |  |  |  | - Materials and tools carried in a backpack or lowered by the Second Technician using a lowering rope. |  |  |
|  |  | Risk of exposure to hazardous/dangerous substances or disease from a build-up of gas discarded needles or sharps, and vermin. | 40 | - If the lift is not a designated Hazardous Goods lift, advise the Building Manager if any suspicious odour or smell is present before entering the shaft pit. | 10 | TKE Personnel |
|  |  |  |  | - Ensure adequate lighting in the shaft and pit and conduct a visual check for evidence of discarded syringes or evidence of vermin in the lift shaft or pit. If identified, report to the Building Manager for removal/treatment. |  |  |
|  |  |  |  | - Report to the site if any evidence of water or inundation before entering the lift pit. |  |  |
| 11 | To conduct the concrete hook (PSDW) test, install the anchor bracket on the concrete structure of the pit on the landing side (the height of the anchor bracket is to be more than 1000mm from the pit floor) | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Conduct a visual check of the condition of the floor and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor during work. |  |  |
| 12 | Attach the hook of the PSDW rope to the machine body of the PSDW (to be completed from the landing floor) | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 7 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
| 13 | Provide temporary power to the PSDW and operate by inches to thread the PSDW rope of the other end through the PDSW | Risk of fall from height from landing into shaft/pit | 50 | - Barrier fixed in place at the entrance to shaft. | 10 | TKE Personnel |
|  |  |  |  | - Barrier to be checked before commencing work |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
| 14 | Install the shackle to the reserved hole on the machine body of the PSDW with a connecting block and fasten it with a bolt. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 10 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |
| 15 | Lift the PSDW slowly to raise it along the rope in the shaft. | Risk of fall from height from landing into shaft/pit | 50 | - Barrier fixed in place at the entrance to shaft. | 10 | TKE Personnel |
|  |  |  |  | - Barrier to be checked before commencing work |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |
| 16 | Install a sling with hooks on both ends under the shackle (maintain SWL of the sling) | Risk of fall from height from landing into shaft/pit | 50 | - Barrier fixed in place at the entrance to shaft. | 10 | TKE Personnel |
|  |  |  |  | - Barrier to be checked before commencing work |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment. |  |  |
| 17 | Connect the sling to the dynamometer with a shackle. Connect the other end of the dynamometer to the anchor bracket with another shackle. | Risk of fall from height from landing into shaft/pit | 50 | - Barrier fixed in place at the entrance to shaft. | 10 | TKE Personnel |
|  |  |  |  | - Barrier to be checked before commencing work. |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment. |  |  |
| 18 | Egress pit | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Before climbing the ladder, conduct a visual check of the condition of the floor and ladder and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
| 19 | Standing on the landing, move the PSDW up and when the dynamometer shows a value of 2T, stop the operation. Hold for at least 5 minutes. Check to make sure the concrete hook and bearing parts are in good condition. Take a photo of the reading. | Risk of fall from height from landing into shaft/pit | 50 | - Barrier fixed in place at the entrance to shaft. | 10 | TKE Personnel |
|  |  |  |  | - Barrier to be checked before commencing work |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |
| 20 | Loosen the rope, removing the test devices. Ensure the PSDW rope in the pit is well protected. | Risk of fall from height from landing into shaft/pit | 50 | - Barrier fixed in place at the entrance to shaft. | 10 | TKE Personnel |
|  |  |  |  | - Barrier to be checked before commencing work. |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |

| # Activity 4 - Template Installation (if using lasers, plumb lines setting with plumb lasers method is applicable to the shaft heights no more than 30 meters) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Job Step Break the job down into logical steps |  | Potential Hazards Identify the hazards associated with each step. Examine each to find possibilities that could lead to an accident. | Initial Risk Level | Controls List the actions necessary to eliminate or minimise the hazards that could lead to an accident, injury, or occupational illness. | Final Risk Level | Person Responsible Identify the Persons responsible for ensuring controls are implemented. |
| 1 | Ensure the top floor barrier is in place and secured before commencing this activity. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
| 2 | Install triangular brackets on both rides of the top landing floor. Fasten with bolts. | Risk of fall from height from landing into shaft/pit | 50 | - Barrier fixed in place at entrance to shaft. | 10 | TKE Personnel |
|  |  |  |  | - Barrier to be checked prior to commencing work. |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 10 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |
| 3 | Mark the door centreline and position of plumb lines on the landing door template rod (from the top landing floor) | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
| 4 | Assemble the landing door template rod and guide rail template rods with the triangular plates, fastening them with bolts. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
| 5 | Install 2 telescopic wall bracing rods on the triangular brackets with slot fasteners (adjust the wall bracing rods to support onto the concrete wall or steel beam of the shaft) | Risk of fall from height from landing into shaft/pit | 50 | - Barrier fixed in place at the entrance to shaft. | 10 | TKE Personnel |
|  |  |  |  | - Barrier to be checked before commencing work |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 10 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |
| 6 | Install the complete template onto the telescopic wall bracing rods with fasteners. | Risk of fall from height from landing into shaft/pit | 50 | - Barrier fixed in place at the entrance to shaft. | 10 | TKE Personnel |
|  |  |  |  | - Barrier to be checked before commencing work |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 10 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |
| 7 | Place the landing door plumb lines into the bottom of the shaft according to the marked position. Measure the shaft on each landing floor to determine and adjust the position of the template accordingly. | Risk of fall from height from landing into shaft/pit | 50 | - Barrier fixed in place at the entrance to shaft. | 10 | TKE Personnel |
|  |  |  |  | - Barrier to be checked before commencing work |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
| 8 | Place the car guide rail plumb lines into the bottom of the shaft (ensuring the plumb lines are in the reserved groove) | Risk of fall from height from landing into shaft/pit | 50 | - Barrier fixed in place at the entrance to shaft. | 10 | TKE Personnel |
|  |  |  |  | - Barrier to be checked before commencing work |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 10 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |
| 9 | Access pit | Risk of medical or another emergency involving a Technician descending the ladder or while working in the pit. | 40 | - Alternative 2-way communication used if visual not available e.g. Technician on Top of Car. | 10 | TKE Personnel |
|  |  |  |  | - “Man Down” feature on Smartphone to be used to alert and notify of medical emergencies. |  |  |
|  |  |  |  | - Emergency Services are to be contacted and the Building Manager is advised in the event of a medical emergency and/or if evacuation from the pit is required. |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Before descending the ladder, conduct a visual check of the condition of the floor and ladder and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
|  |  |  |  | - Ladder to be secured |  |  |
|  |  | Risk of injury from Falling Objects – objects falling from landing onto the Technician in the Pit. | 28 | - No items stored in the vicinity of the edge when doors open. | 14 | TKE Personnel |
|  |  |  |  | - Landing doors closed to a maximum width of 200mm once Technician One has entered the pit. |  |  |
|  |  |  |  | - Materials and tools carried in a backpack or lowered by a Second Technician using a lowering rope. |  |  |
|  |  | Risk of exposure to hazardous/dangerous substances or disease from a build-up of gas discarded needles or sharps, and vermin. | 40 | - If the lift is not a designated Hazardous Goods lift, advise the Building Manager if any suspicious odour or smell is present before entering the shaft pit. | 10 | TKE Personnel |
|  |  |  |  | - Ensure adequate lighting in the shaft and pit and conduct a visual check for evidence of discarded syringes or evidence of vermin in the lift shaft or pit. If identified, report to the Building Manager for removal/treatment. |  |  |
|  |  |  |  | - Report to the site if any evidence of water or inundation before entering the lift pit. |  |  |
| 10 | Install the lower templates in the pit – hang the weights under the plumb lines and immerse them into water or oil buckets. Fix the plumb lines on the lower templates with a short rope. Check the spacing between all the relevant plumb lines. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Conduct a visual check of the condition of the floor and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 10 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |
| 11 | Egress pit | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Before climbing the ladder, conduct a visual check of the condition of the floor and ladder and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
|  |  |  |  | - Ladder to be secured. |  |  |

| # Activity 5 - Pit Components Installation |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Job Step Break the job down into logical steps |  | Potential Hazards Identify the hazards associated with each step. Examine each to find possibilities that could lead to an accident. | Initial Risk Level | Controls List the actions necessary to eliminate or minimise the hazards that could lead to an accident, injury, or occupational illness. | Final Risk Level | Person Responsible Identify the Persons responsible for ensuring controls are implemented. |
| 1 | Access pit | Risk of medical or another emergency involving a technician descending the ladder or while working in the pit. | 40 | - Alternative 2-way communication used if visual not available e.g. Technician on Top of Car. | 10 | TKE Personnel |
|  |  |  |  | - “Man Down” feature on Smartphone to be used to alert and notify of medical emergencies. |  |  |
|  |  |  |  | - Emergency Services are to be contacted and the Building Manager is advised in the event of a medical emergency and/or if evacuation from the pit is required. |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Before descending the ladder, conduct a visual check of the condition of the floor and ladder and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
|  |  |  |  | - Ladder to be secured. |  |  |
|  |  | Risk of injury from Falling Objects – objects falling from landing onto the Technician in the Pit. | 28 | - No items stored in the vicinity of the edge when doors open. | 14 | TKE Personnel |
|  |  |  |  | - Landing doors closed to a maximum width of 200mm once Technician One has entered the pit. |  |  |
|  |  |  |  | - Materials and tools carried in a backpack or lowered by a Second Technician using a lowering rope. |  |  |
|  |  | Risk of exposure to hazardous/dangerous substances or disease from a build-up of gas discarded needles or sharps, and vermin. | 40 | - If the lift is not a designated Hazardous Goods lift, advise the Building Manager if any suspicious odour or smell is present before entering the shaft pit. | 10 | TKE Personnel |
|  |  |  |  | - Ensure adequate lighting in the shaft and pit and conduct a visual check for evidence of discarded syringes or evidence of vermin in the lift shaft or pit. If identified, report to the Building Manager for removal/treatment. |  |  |
|  |  |  |  | - Report to the site if any evidence of water or inundation before entering the lift pit. |  |  |
| 2 | Install first-level car guide rail brackets and counterweight guide rail brackets in the pit. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Conduct a visual check of the condition of the floor and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 10 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |
| 3 | Install the first car guide rails on the rail base and adjust accordingly. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Conduct a visual check of the condition of the floor and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 10 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |
| 4 | Using the PSDW, lift the bottom beam of the car sling into the pit (from the bottom landing). Use 4 adjustable base jacks evenly at the 4 corners of the bottom beam and fasten with bolts. Level the bottom beam by adjusting the nuts. | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Conduct a visual check of the condition of the floor and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
|  |  | Risk of fall from height from landing into shaft/pit | 50 | - Barrier fixed in place at the entrance to shaft. | 10 | TKE Personnel |
|  |  |  |  | - Barrier to be checked before commencing work. |  |  |
|  |  | Risk of falling objects | 40 | - No personnel are to be in the lift shaft when a part is being moved. | 10 | TKE Personnel |
|  |  |  |  | - No personnel to work above or below the lift car. |  |  |
| 5 | Install the guide rail shoes on both sides of the bottom beam of the car sling. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Conduct a visual check of the condition of the floor and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 10 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |
| 6 | Install scaffold board onto the bottom beam. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Conduct a visual check of the condition of the floor and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
| 7 | Install the 2nd level car rail brackets and counterweight rail bracket (place the telescopic ladder onto the car bottom) | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
| 8 | Install the counterweight guide rail on the two-in-one bracket. Install the counterweight guide rail on the other side. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
| 9 | Install the counterweight buffer and pit ladder. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
| 10 | Egress pit | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Before climbing the ladder, conduct a visual check of the condition of the floor and ladder and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
|  |  |  |  | - Ladder to be secured |  |  |

| # Activity 6 - Counterweight Frame Installation |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Job Step Break the job down into logical steps |  | Potential Hazards Identify the hazards associated with each step. Examine each to find possibilities that could lead to an accident. | Initial Risk Level | Controls List the actions necessary to eliminate or minimise the hazards that could lead to an accident, injury, or occupational illness. | Final Risk Level | Person Responsible Identify the Persons responsible for ensuring controls are implemented. |
| 1 | Using the installed rigging gear in the shaft, lift the counterweight frame from the landing. | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of fall from height from landing into shaft/pit | 50 | - Stay clear of the landing edge when moving equipment. | 10 | TKE Personnel |
|  |  |  |  | - Where not possible, fall protection to be worn |  |  |
|  |  | Risk of falling objects | 40 | - No personnel are to be in the lift shaft when a part is being moved | 10 | TKE Personnel |
|  |  |  |  | - No personnel to work above or below the lift car. |  |  |
| 2 | Access pit | Risk of medical or another emergency involving a Technician descending the ladder or while working in the pit. | 40 | - Alternative 2-way communication used if visual not available e.g. Technician on Top of Car. | 10 | TKE Personnel |
|  |  |  |  | - “Man Down” feature on Smartphone to be used to alert and notify of medical emergency. |  |  |
|  |  |  |  | - Emergency Services are to be contacted and the Building Manager is advised in the event of a medical emergency and/or if evacuation from the pit is required. |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Before descending the ladder, conduct a visual check of the condition of the floor and ladder and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
|  |  |  |  | - Ladder to be secured. |  |  |
|  |  | Risk of injury from Falling Objects – objects falling from landing onto the Technician in the Pit. | 28 | - No items stored in the vicinity of the edge when doors open. | 14 | TKE Personnel |
|  |  |  |  | - Landing doors closed to a maximum width of 200mm once Technician One has entered the pit. |  |  |
|  |  |  |  | - Materials and tools carried in a backpack or lowered by a Second Technician using a lowering rope. |  |  |
|  |  | Risk of exposure to hazardous/dangerous substances or disease from a build-up of gas discarded needles or sharps, and vermin. | 40 | - If the lift is not a designated Hazardous Goods lift, advise the Building Manager if any suspicious odour or smell is present before entering the shaft pit. | 10 | TKE Personnel |
|  |  |  |  | - Ensure adequate lighting in the shaft and pit and conduct a visual check for evidence of discarded syringes or evidence of vermin in the lift shaft or pit. If identified, report to the Building Manager for removal/treatment. |  |  |
|  |  |  |  | - Report to the site if any evidence of water or inundation before entering the lift pit. |  |  |
| 3 | Position into place and secure. | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 10 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |
| 4 | Place a wooden plank on the pit floor with 4 guide rails and 4 counterweight rails vertically in the shaft. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Conduct a visual check of the condition of the floor and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
| 5 | Egress pit | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Before climbing the ladder, conduct a visual check of the condition of the floor and ladder and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
|  |  |  |  | - Ladder to be secured. |  |  |

| # Activity 7 - Car Sling and Cabin Installation |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Job Step Break the job down into logical steps |  | Potential Hazards Identify the hazards associated with each step. Examine each to find possibilities that could lead to an accident. | Initial Risk Level | Controls List the actions necessary to eliminate or minimise the hazards that could lead to an accident, injury, or occupational illness. | Final Risk Level | Person Responsible Identify the Persons responsible for ensuring controls are implemented. |
| 1 | Using rigging gear installed in the shaft, hoist the car bottom, car sling, and cabin equipment into the shaft | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of fall from height from landing into shaft/pit | 50 | - Stay clear of the landing edge when moving equipment | 10 | TKE Personnel |
|  |  |  |  | - Where not possible, fall protection to be worn |  |  |
|  |  | Risk of falling objects | 40 | - No personnel are to be in the lift shaft when a part is being moved. | 10 | TKE Personnel |
|  |  |  |  | - No personnel to work above or below the lift car. |  |  |
| 2 | Install upper beam into place | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always maintain three points of contact | 10 | TKE Personnel |
|  |  |  |  | - Platform ladder or mobile scaffold is to be used for securing bolts |  |  |
|  |  | Risk of falling objects | 40 | - No personnel are to be in the lift shaft when a part is being moved. | 10 | TKE Personnel |
|  |  |  |  | - No personnel to work above or below the lift car. |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 10 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |
| 3 | Install upper guide shoe | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always maintain three points of contact | 10 | TKE Personnel |
|  |  |  |  | - Platform ladder or mobile scaffold is to be used for securing. |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 10 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |
| 4 | Adjust clearance of safety gear. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always maintain three points of contact. | 10 | TKE Personnel |
|  |  |  |  | - Platform ladder or mobile scaffold is to be used for securing parts. |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 10 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |
| 5 | Remove the scaffold/deck used as the working platform | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
| 6 | Install the car bottom using rigging gear. | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of fall from height from landing into shaft/pit | 50 | - Stay clear of the landing edge when moving equipment. | 10 | TKE Personnel |
|  |  |  |  | - Where not possible, fall protection to be worn. |  |  |
|  |  | Risk of falling objects | 40 | - No personnel are to be in the lift shaft when a part is being moved. | 10 | TKE Personnel |
|  |  |  |  | - No personnel to work above or below the lift car. |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 10 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |
| 7 | Lift the car roof using the PSDW and 2nd anchor point to the bottom of the car's upper beam (ensuring there is space for car wall installation) | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of fall from height from landing into shaft/pit | 50 | - Stay clear of the landing edge when moving equipment. | 10 | TKE Personnel |
|  |  |  |  | - Where not possible, fall protection to be worn |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 10 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |
| 8 | Install the car walls and car roof balustrade (guide rails) | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always maintain three points of contact | 10 | TKE Personnel |
|  |  |  |  | - Platform ladder or mobile scaffold is to be used for securing parts. |  |  |
|  |  | Risk of falling objects | 40 | - No personnel are to be in the lift shaft when a part is being moved. | 10 | TKE Personnel |
|  |  |  |  | - No personnel to work above or below the lift car. |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 10 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |

| # Activity 8 - Car Top Hoisting Assembly Installation |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Job Step Break the job down into logical steps |  | Potential Hazards Identify the hazards associated with each step. Examine each to find possibilities that could lead to an accident. | Initial Risk Level | Controls List the actions necessary to eliminate or minimise the hazards that could lead to an accident, injury, or occupational illness. | Final Risk Level | Person Responsible Identify the Persons responsible for ensuring controls are implemented. |
| 1 | Install the connecting base using the PSDW. (Connecting Base I (PSDW) into the car upper beam installation process hole and Connecting Base II (PSDW rope hook) into the opposite car upper beam installation process hole) | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of fall from height | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  | Risk of falling objects | 40 | - No personnel to work above or below the lift car. | 10 | TKE Personnel |
|  |  |  |  | - Keep the top of the car free of excess material, debris, etc |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 10 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |
| 2 | Ensure all cotter pins are correct and secure. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
| 3 | Install the PSDW and lifting lug. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
| 4 | Place a wooden plate on the car's upper beam hoist, jog PSDW, and remove the PSDW hook. Connect it to Connecting Base II on the car's upper beam. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling objects | 40 | - No personnel to work above or below the lift car. | 10 | TKE Personnel |
|  |  |  |  | - Keep the top of the car free of excess material, debris, etc |  |  |
| 5 | Install the car 2nd protection – use shackles to connect the car 2nd protection steel wire rope with the reserved installation holes on Connection Base I & II respectively. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
| 6 | 1) Install the crossbar. 2) Place wood at the base of each bar and fix the crossbars to the car balustrades. 3) Install the shorter crossbars on the far side of the shaft so the higher side is at the landing door side. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of fall from height | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  | Risk of falling objects | 40 | - No personnel to work above or below the lift car. | 10 | TKE Personnel |
|  |  |  |  | - Keep the top of the car free of excess material, debris, etc |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 10 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |
| 7 | On the Governor's side, install the blocstop fixings pole. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of fall from height | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  | Risk of falling objects | 40 | - No personnel to work above or below the lift car. | 10 | TKE Personnel |
|  |  |  |  | - Keep the top of the car free of excess material, debris, etc |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 10 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |
| 8 | Install the horizontal and vertical rods of the car overhead protection. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling objects | 40 | - No personnel to work above or below the lift car. | 10 | TKE Personnel |
|  |  |  |  | - Keep the top of the car free of excess material, debris, etc |  |  |
|  |  | Risk of fall from heights from the top of the car. | 50 | - Always keep within the perimeter of the lift car top handrail. | 6 | TKE Personnel |
|  |  |  |  | - Use fall protection equipment if working above the guardrail on a platform ladder or scaffold. |  |  |
|  |  |  |  | - Lift car not to be moved during work. |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 10 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |
| 9 | Install the car overhead protection plate (ensuring no interference with the steel wire ropes) | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling objects | 40 | - No personnel to work above or below the lift car. | 10 | TKE Personnel |
|  |  |  |  | - Keep the top of the car free of excess material, debris, etc |  |  |
|  |  | Risk of fall from heights from the top of the car. | 50 | - Always keep within the perimeter of the lift car top handrail. | 6 | TKE Personnel |
|  |  |  |  | - Use fall protection equipment if working above the guardrail on a platform ladder or scaffold. |  |  |
|  |  |  |  | - Lift car not to be moved during work. |  |  |
| 10 | Install the blocstop trigger mechanism with its fixing pole and insert the blocstop steel wire rope. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling objects | 40 | - No personnel to work above or below the lift car. | 10 | TKE Personnel |
|  |  |  |  | - Keep the top of the car free of excess material, debris, etc |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. | 10 | TKE Personnel |
| 11 | Install the anti-derailment limit switch (adjust position accordingly) | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling objects | 40 | - No personnel to work above or below the lift car. | 10 | TKE Personnel |
|  |  |  |  | - Keep the top of the car free of excess material, debris, etc |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 10 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |

| # Activity 9 - Blocstop-Safety Gear Activation Test |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Job Step Break the job down into logical steps |  | Potential Hazards Identify the hazards associated with each step. Examine each to find possibilities that could lead to an accident. | Initial Risk Level | Controls List the actions necessary to eliminate or minimise the hazards that could lead to an accident, injury, or occupational illness. | Final Risk Level | Person Responsible Identify the Persons responsible for ensuring controls are implemented. |
| 1 | Connect the block top and safety gear lifting mechanism with a short sling. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling objects | 40 | - No personnel to work above or below the lift car. | 10 | TKE Personnel |
|  |  |  |  | - Keep the top of the car free of excess material, debris, etc |  |  |
| 2 | Operate the PSDW upwards to make sure there will be enough lifting space for the block top. Manually lock the block top and then operate the PSDW downwards slowly until the PSDW rope loosens. | Risk of fall from height from top of car into shaft/pit | 50 | - Handrails in place on top of the car where the gap between the shaft wall and outside perimeter exceeds 300mm. If none is in place, alternative fall protection is to be used. Harnesses should not be used when the lift car is moving due to the possibility of entanglement. The lift car should be secured and stopped from moving before use. | 10 | TKE Personnel |
|  |  |  |  | - Maintain all body within the limits of the lift car top whilst traveling. |  |  |
|  |  |  |  | - Ensure kickboards and handrails are correctly assembled. |  |  |
|  |  | Risk of crush injury from uncontrolled movement of lift car or other crush injury when moving lift car | 50 | - Obtain acknowledgment from other technicians before moving the lift car by SWP 10 (if other personnel are on the lift car). | 10 | TKE Personnel |
|  |  |  |  | - Ensure when moving the car that all materials and occupants remain within the confines of the lift car perimeter. |  |  |
| 3 | Operate the PSDW upwards until the PSDW rope is tensioned and safety gears are released from the guide rail. | Risk of fall from height from top of car into shaft/pit | 50 | - Handrails in place on top of the car where the gap between the shaft wall and outside perimeter exceeds 300mm. If none is in place, alternative fall protection is to be used. Harnesses should not be used when the lift car is moving due to the possibility of entanglement. The lift car should be secured and stopped from moving before use. | 10 | TKE Personnel |
|  |  |  |  | - Maintain all body within the limits of the lift car top whilst traveling. |  |  |
|  |  |  |  | - Ensure kickboards and handrails are correctly assembled. |  |  |
|  |  | Risk of crush injury from uncontrolled movement of lift car or other crush injury when moving lift car | 50 | - Obtain acknowledgment from other technicians before moving the lift car per SWP 10 (if other personnel are on the lift car). | 10 | TKE Personnel |
|  |  |  |  | - Ensure when moving the car that all materials and occupants remain within the confines of the lift car perimeter. |  |  |
| 4 | Manually unlock the blocstop | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |

| # Activity 10 - Other Special Toolings Installation (Guide Devices and Fixing Fixture Installation and Rail Transport Toolings Installation) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Job Step Break the job down into logical steps |  | Potential Hazards Identify the hazards associated with each step. Examine each to find possibilities that could lead to an accident. | Initial Risk Level | Controls List the actions necessary to eliminate or minimise the hazards that could lead to an accident, injury, or occupational illness. | Final Risk Level | Person Responsible Identify the Persons responsible for ensuring controls are implemented. |
| 1 | Move the car up the lift shaft and install the car buffer in the pit. | Risk of medical or another emergency involving a technician descending the ladder or while working in the pit. | 40 | - Alternative 2-way communication used if visual is not available. | 10 | TKE Personnel |
|  |  |  |  | - “Man Down” feature on Smartphone to be used to alert and notify of medical emergencies. |  |  |
|  |  |  |  | - Emergency Services are to be contacted and the Building Manager is advised in the event of a medical emergency and/or if evacuation from the pit is required. |  |  |
|  |  | Risk of Fall Hazard entering the Shaft to descend the ladder to the Pit. | 50 | - Confirm the ladder within 1200mm from the edge of the landing door. If not, alternative fall protection may be required. | 10 | TKE Personnel |
|  |  |  |  | - Lighting in Lift Shaft switched on and working and adequate to sight ladder. |  |  |
|  |  |  |  | - Confirm ladder appears in good working order and is not covered in any liquid/material that might result in a slip. |  |  |
|  |  |  |  | - 3 points of contact to be always maintained while descending or ascending the ladder. Equipment, tools, or materials are not to be carried in hands while on the ladder. All Equipment is to be carried in a backpack. |  |  |
|  |  |  |  | - Other material/equipment not carried in backpack lowered into pit via alternative means (lowering rope). |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Before descending the ladder, conduct a visual check of the condition of the floor and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor during servicing. |  |  |
|  |  | Risk of injury from Falling Objects – objects falling from landing onto the Technician in the Pit. | 28 | - No items stored in the vicinity of the landing edge. | 14 | TKE Personnel |
|  |  |  |  | - Materials and tools carried in a backpack or lowered by the Second Technician using a lowering rope. |  |  |
|  |  | Risk of exposure to hazardous/dangerous substances or disease from build-up of gas discarded needles or sharps, and vermin. | 40 | - If the lift is not a designated Hazardous Goods lift, advise the Building Manager if any suspicious odour or smell is present before entering the shaft pit. | 10 | TKE Personnel |
|  |  |  |  | - Ensure adequate lighting in the shaft and pit and conduct a visual check for evidence of discarded syringes or evidence of vermin in the lift shaft or pit. If identified, report to the Building Manager for removal/treatment. |  |  |
|  |  |  |  | - Report to the site if any evidence of water or inundation before entering the lift pit. |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of eye injury, ear injury & dust contamination from concrete drilling | 28 | - Wear eye protection, ear protection, and dust mask. | 14 | TKE Personnel |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 10 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |
| 2 | Remove the adjustable base jacks from the bottom beam of the car sling. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
| 3 | Hang a weight under the blocstop rope and fasten it with bolts. Install the tensioner of the overspeed governor in the pit. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Conduct a visual check of the condition of the floor and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
| 4 | Under the car (the bottom beam), install the guide device of the PSDW rope on the PSDW side, near the landing door side. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Conduct a visual check of the condition of the floor and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
| 5 | Install the guide device on the pit floor. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. - Where this is not possible, use safe lifting principles or two-man lifts. - Ensure work areas and fire exits are clear of material. | 14 | TKE Personnel |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Conduct a visual check of the condition of the floor and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
| 6 | Pull the hoist machine rope through the guide pulley | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
| 7 | Install the fixing fixture of the PSDW rope to the bottom beam with a short sling and fasten the PSDW rope to the fixture with a fixture block. Coil and hang the excess PSDW rope and fix the rope temporarily. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Conduct a visual check of the condition of the floor and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
| 8 | Install the lower rail transport bracket and upper rail transport bracket on the landing side of the car. Tighten with bolts. Do not drill holes or modify the brackets. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Conduct a visual check of the condition of the floor and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 10 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |
| 9 | Using the lifting gear in the shaft, move 4 rails sitting at the bottom of the shaft into the rail transport brackets. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
| 10 | Egress pit | Risk of medical or another emergency involving a Technician ascending the ladder or while working in the pit. | 40 | - 2-way communication used if visual is not available | 10 | TKE Personnel |
|  |  |  |  | - “Man Down” feature on Smartphone to be used to alert and notify of medical emergencies. |  |  |
|  |  |  |  | - Emergency Services are to be contacted and the Building Manager is advised in the event of a medical emergency and/or if evacuation from the pit is required. |  |  |
|  |  | Risk of Fall Hazard to First Technician using a ladder to exit. | 40 | - Confirm ladder is within 1200mm from the edge of the landing door. If not, alternative fall protection may be required. | 10 | TKE Personnel |
|  |  |  |  | - Lighting in Lift Shaft switched on and working and adequate to sight ladder. |  |  |
|  |  |  |  | - Confirm ladder appears in good working order and is not covered in any liquid/material that might result in a slip. |  |  |
|  |  |  |  | - 3 points of contact to be maintained at all times while descending or ascending the ladder. Equipment, tools, or materials are not to be carried in hands while on the ladder. All Equipment is to be carried in a backpack. |  |  |
|  |  |  |  | - Other material/equipment not carried in backpack raised out of pit via alternative means (rope). |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Before ascending the ladder, conduct a visual check of the condition of the floor and ladder and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor during work. |  |  |
|  |  | Risk of injury from Falling Objects – objects falling from landing onto Technician. | 28 | - No items stored in the vicinity of the landing edge. | 14 | TKE Personnel |

| # Activity 11 - Remaining Guide Rails Installation (Guide Rails and Counterweight Rails) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Job Step Break the job down into logical steps |  | Potential Hazards Identify the hazards associated with each step. Examine each to find possibilities that could lead to an accident. | Initial Risk Level | Controls List the actions necessary to eliminate or minimise the hazards that could lead to an accident, injury, or occupational illness. | Final Risk Level | Person Responsible Identify the Persons responsible for ensuring controls are implemented. |
| 1 | Install the rail brackets on both sides of the shaft from the top of the car | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  |  |  | - Use fall protection equipment if working above the guardrail on a platform ladder or scaffold. |  |  |
|  |  |  |  | - Lift car not to be moved during works |  |  |
|  |  | Risk of falling objects | 40 | - No personnel are to be in the lift shaft when a part is being moved. | 10 | TKE Personnel |
|  |  |  |  | - No personnel to work above or below the lift car. |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 10 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |
| 2 | With the safety gears engaged, lock the blocstop manually and move the car downwards slowly until the PSDW rope loosens and slacks. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
| 3 | Remove one bolt from the fishplate of the installed guide rail and install the secondary protection rope. Repeat for the other side. | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials / equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
| 4 | Detach the hook of PSDW rope from the connecting base | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
| 5 | Using the guide rail lifting fixture and the hook of the PSDW rope, move the guide rail and secure it above the bottom rail. | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
| 6 | Reattach the hook of the PSDW rope to the connecting base and move the car upwards slowly until the car is secured in the devices. | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
| 7 | Unlock the block top and remove the secondary protection device (placing it on the top of the car) | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
| 8 | Move the car upwards and repeat steps 2 – 7 | As above |  |  |  | TKE Personnel |
| 9 | Remove the template and plumb lines from the top landing (top floor barrier to be removed after the template and plumb lines are removed) | Risk of fall from height from landing into shaft/pit | 50 | - Barrier fixed in place at the entrance to shaft. | 10 | TKE Personnel |
|  |  |  |  | - Barrier to be checked before commencing work. |  |  |
|  |  |  |  | - The top of the car to be accessible at the landing level |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
| 10 | Install the top rail brackets, guide rails, and the base of the overspeed governor to the top of the guide rail. (for Meta 100 this is to be installed after the installation of the traction machine in Section 13) | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
| 11 | Place the counterweight guide rails into the rail transport brackets | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
| 12 | Install counterweight brackets onto the shaft wall | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
| 13 | With the safety gears engaged, lock the blocstop manually and move the car downwards slowly until the PSDW rope loosens and slacks. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
| 14 | Remove one bolt from the fishplate of the installed guide rail and install the secondary protection rope. Repeat for the other side. | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment. |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
| 15 | Detach the hook of the PSDW rope from the connecting base. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
| 16 | Using the counterweight rail lifting fixture and the hook of the PSDW rope, move the counterweight rail and secure it above the bottom rail | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
| 17 | Place hook back onto connecting base and move car upwards slowly, remove secondary protection device. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
| 18 | Move the car upwards and repeat steps 12 – 17 | As above |  |  |  | TKE Personnel |

| # Activity 12 Landing Doors Installation |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Job Step Break the job down into logical steps |  | Potential Hazards Identify the hazards associated with each step. Examine each to find possibilities that could lead to an accident. | Initial Risk Level | Controls List the actions necessary to eliminate or minimise the hazards that could lead to an accident, injury, or occupational illness. | Final Risk Level | Person Responsible Identify the Persons responsible for ensuring controls are implemented. |
| 1 | From the top of car, install landing doors on each floor (including headers, unlocking devices, and sill) The top-floor landing doors are to be installed after the traction machine is installed. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  |  |  | - Use fall protection equipment if working above the guardrail on a platform ladder or scaffold. |  |  |
|  |  |  |  | - Lift car not to be moved during work. |  |  |
| 2 | Adjust the doors and check for clearances. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  |  |  | - Use fall protection equipment if working above the guardrail on a platform ladder or scaffold. |  |  |
|  |  |  |  | - Lift car not to be moved during works |  |  |

| # Activity 13 - Overhead Equipment Installation |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Job Step Break the job down into logical steps |  | Potential Hazards Identify the hazards associated with each step. Examine each to find possibilities that could lead to an accident. | Initial Risk Level | Controls List the actions necessary to eliminate or minimise the hazards that could lead to an accident, injury, or occupational illness. | Final Risk Level | Person Responsible Identify the Persons responsible for ensuring controls are implemented. |
| 1 | Remove car top protection plates, supporting rods of protection plates, and part of uprights. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  |  |  | - Use fall protection equipment if working above the guardrail on a platform ladder or scaffold. |  |  |
|  |  |  |  | - Lift car not to be moved during work. |  |  |
| 2 | Move the car to the top of shaft and hang 2 x 2T chain blocks onto hooks/anchor points at the top of shaft | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  |  |  | - Use fall protection equipment if working above the guardrail on a platform ladder or scaffold. |  |  |
|  |  |  |  | - Lift car not to be moved during work. |  |  |
| 3 | Drill into the shaft wall and install the machine beam. | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  |  |  | - Use fall protection equipment if working above the guardrail on a platform ladder or scaffold. |  |  |
|  |  |  |  | - Lift car not to be moved during work. |  |  |
|  |  |  |  | - Keep the top of car free of excess material, debris, etc |  |  |
| 4 | Install lifting plate to the machine. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  |  |  | - Use fall protection equipment if working above the guardrail on a platform ladder or scaffold. |  |  |
|  |  |  |  | - Lift car not to be moved during work. |  |  |
|  |  |  |  | - Keep the top of the car free of excess material, debris, etc |  |  |
| 5 | Using the lifting gear, hoist the traction machine into the shaft | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
| 6 | Place and fix the machine on the machine beam (arrange the vibration absorbers between the machine and the beam) | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  |  |  | - Use fall protection equipment if working above the guardrail on a platform ladder or scaffold. |  |  |
|  |  |  |  | - Lift car not to be moved during work. |  |  |
|  |  |  |  | - Keep the top of the car free of excess material, debris, etc |  |  |
| 7 | Install the top guide rail of the machine side and the damper. | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  |  |  | - Use fall protection equipment if working above the guardrail on a platform ladder or scaffold. |  |  |
|  |  |  |  | - Lift car not to be moved during work. |  |  |
|  |  |  |  | - Keep the top of the car free of excess material, debris, etc |  |  |
| 8 | Install the beam of the overspeed governor and top guide rail for this side. | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  |  |  | - Use fall protection equipment if working above the guardrail on a platform ladder or scaffold. |  |  |
|  |  |  |  | - Lift car not to be moved during work. |  |  |
|  |  |  |  | - Keep the top of car free of excess material, debris, et |  |  |
| 9 | Install the overspeed governor on the beam. | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  |  |  | - Use fall protection equipment if working above the guardrail on a platform ladder or scaffold. |  |  |
|  |  |  |  | - Lift car not to be moved during work. |  |  |
|  |  |  |  | - Keep the top of the car free of excess material, debris, etc |  |  |

| # Activity 14 - Lift Shaft Electrical Components Installation |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Job Step Break the job down into logical steps |  | Potential Hazards Identify the hazards associated with each step. Examine each to find possibilities that could lead to an accident. | Initial Risk Level | Controls List the actions necessary to eliminate or minimise the hazards that could lead to an accident, injury, or occupational illness. | Final Risk Level | Person Responsible Identify the Persons responsible for ensuring controls are implemented. |
| 10 | Install controller cabinet and rescue tool storage box on top landing. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 10 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |
|  |  | Risk of eye injury, ear injury & dust contamination from concrete drilling | 28 | - Wear eye protection, ear protection, and dust mask. | 14 | TKE Personnel |
|  |  | Risk of falling objects | 40 | No work is to be completed in the lift shaft or the pit | 10 | TKE Personnel |
| 11 | Install a resistor box from the top of the car. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  |  |  | - Lift car not to be moved during work. |  |  |
|  |  |  |  | - Keep the top of the car free of excess material, debris, etc |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 10 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |
| 12 | Install overhead wiring. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  |  |  | - Use fall protection equipment if working above the guardrail on a platform ladder or scaffold. |  |  |
|  |  |  |  | - Lift car not to be moved during work. |  |  |
|  |  |  |  | - Keep the top of the car free of excess material, debris, etc |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 10 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |
| 13 | Complete the motor test. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of crush injury from uncontrolled movement of lift car or other crush injury when moving lift car | 50 | - Obtain acknowledgment from other technicians before moving the lift car per SWP 10 (if other personnel are on the lift car). | 10 | TKE Personnel |
|  |  |  |  | - Ensure when moving the car that all materials and occupants remain within the confines of the lift car perimeter. |  |  |
| 14 | Place the traveling cable reel (including brackets for the reel) on top of the car. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  |  |  | - Lift car not to be moved during works |  |  |
|  |  |  |  | - Keep the top of the car free of excess material, debris, etc |  |  |
| 15 | Pull out one end of the traveling cable and connect it to the controller cabinet through the shaft wall. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  |  |  | - Use fall protection equipment if working above the guardrail on a platform ladder or scaffold. |  |  |
|  |  |  |  | - Lift car not to be moved during work. |  |  |
|  |  |  |  | - Keep the top of the car free of excess material, debris, etc |  |  |
| 16 | Install cable brackets into the shaft wall to support the traveling cable. Arrange and fix the cable. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  |  |  | - Use fall protection equipment if working above a guardrail on a platform ladder or scaffold |  |  |
|  |  |  |  | - Lift car not to be moved during work. |  |  |
|  |  |  |  | - Keep the top of the car free of excess material, debris, etc |  |  |
| 17 | Move the car downwards slowly and release more traveling cable. Install cable brackets along the shaft and fix the cable. Fix any cable twists along the way, | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of crush injury from uncontrolled movement of lift car or other crush injury when moving lift car | 50 | - Obtain acknowledgment from other technicians before moving the lift car per SWP 10 (if other personnel are on the lift car). | 10 | TKE Personnel |
|  |  |  |  | - Ensure when moving the car that all materials and occupants remain within the confines of the lift car perimeter. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  |  |  | - Keep the top of the car free of excess material, debris, etc |  |  |
| 18 | Access pit | Risk of medical or another emergency involving a Technician descending the ladder or while working in the pit. | 40 | - Alternative 2-way communication used if visual not available e.g. Technician on Top of Car. | 10 | TKE Personnel |
|  |  |  |  | - “Man Down” feature on Smartphone to be used to alert and notify of medical emergencies. |  |  |
|  |  |  |  | - Emergency Services are to be contacted and the Building Manager is advised in the event of a medical emergency and/or if evacuation from the pit is required. |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Before descending the ladder, conduct a visual check of the condition of the floor and ladder and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
|  |  | Risk of injury from Falling Objects – objects falling from landing onto the Technician in the Pit. | 28 | - No items stored in the vicinity of the edge when doors open. | 14 | TKE Personnel |
|  |  |  |  | - Landing doors closed to a maximum width of 200mm once Technician One has entered the pit. |  |  |
|  |  |  |  | - Materials and tools carried in a backpack or lowered by the Second Technician using a lowering rope. |  |  |
|  |  | Risk of exposure to hazardous/dangerous substances or disease from a build-up of gas discarded needles or sharps, and vermin. | 40 | - If the lift is not a designated Hazardous Goods lift, advise the Building Manager if any suspicious odour or smell is present before entering the shaft pit. | 10 | TKE Personnel |
|  |  |  |  | - Ensure adequate lighting in the shaft and pit and conduct a visual check for evidence of discarded syringes or evidence of vermin in the lift shaft or pit. If identified, report to the Building Manager for removal/treatment. |  |  |
|  |  |  |  | - Report to the site if any evidence of water or inundation before entering the lift pit. |  |  |
| 19 | When at the bottom of the shaft, install the cable bracket to the bracket on the car bottom. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Conduct a visual check of the condition of the floor and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 10 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |
| 20 | Egress pit | Risk of medical or another emergency involving a Technician ascending the ladder or while working in the pit. | 40 | - 2-way communication used if visual is not available | 10 | TKE Personnel |
|  |  |  |  | - “Man Down” feature on Smartphone to be used to alert and notify of medical emergencies. |  |  |
|  |  |  |  | - Emergency Services are to be contacted and the Building Manager is advised in the event of a medical emergency and/or if evacuation from the pit is required. |  |  |
|  |  | Risk of Fall Hazard to First Technician using a ladder to exit. | 40 | - Confirm the ladder within 1200mm from the edge of the landing door. If not, alternative fall protection may be required. | 10 | TKE Personnel |
|  |  |  |  | - Lighting in Lift Shaft switched on and working and adequate to sight ladder. |  |  |
|  |  |  |  | - Confirm ladder appears in good working order and is not covered in any liquid/material that might result in a slip. |  |  |
|  |  |  |  | - 3 points of contact to be always maintained while descending or ascending the ladder. Equipment, tools, or materials are not to be carried in hands while on the ladder. All Equipment is to be carried in a backpack. |  |  |
|  |  |  |  | - Other material/equipment not carried in backpack raised out of pit via alternative means (rope). |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Before ascending the ladder, conduct a visual check of the condition of the floor and ladder and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor during work. |  |  |
|  |  | Risk of injury from Falling Objects – objects falling from landing onto Technician. | 28 | - No items stored in the vicinity of the landing edge. | 14 | TKE Personnel |
| 21 | Access the top of the car temporarily fix the inspection box to the car top and connect the cable end to the inspection box on the car top. | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of crush injury from uncontrolled movement of lift car or other crush injury when moving lift car | 50 | - Obtain acknowledgment from other technicians before moving the lift car per SWP 10 (if other personnel are on the lift car). | 10 | TKE Personnel |
|  |  |  |  | - Ensure when moving the car that all materials and occupants remain within the confines of the lift car perimeter. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  |  |  | - Keep the top of the car free of excess material, debris, etc |  |  |

| # Activity 15 - Traction Machine Roping |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Job Step Break the job down into logical steps |  | Potential Hazards Identify the hazards associated with each step. Examine each to find possibilities that could lead to an accident. | Initial Risk Level | Controls List the actions necessary to eliminate or minimise the hazards that could lead to an accident, injury, or occupational illness. | Final Risk Level | Person Responsible Identify the Persons responsible for ensuring controls are implemented. |
| 1 | Position the car at the bottom of the shaft and install the diversion pulley | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of crush injury from uncontrolled movement of lift car or other crush injury when moving lift car | 50 | - Obtain acknowledgment from other technicians before moving the lift car per SWP 10 (if other personnel are on the lift car). | 10 | TKE Personnel |
|  |  |  |  | - Ensure when moving the car that all materials and occupants remain within the confines of the lift car perimeter. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  |  |  | - Keep the top of the car free of excess material, debris, etc |  |  |
| 2 | Place traction rope reels on top of the car. | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of crush injury from uncontrolled movement of lift car or other crush injury when moving lift car | 50 | - Obtain acknowledgment from other technicians before moving the lift car per SWP 10 (if other personnel are on the lift car). | 10 | TKE Personnel |
|  |  |  |  | - Ensure when moving the car that all materials and occupants remain within the confines of the lift car perimeter. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  |  |  | - Keep the top of the car free of excess material, debris, etc |  |  |
| 3 | On the overspeed governor side, lower the traction rope to the car bottom. Pass the rope through the rope groove of the diversion pulley at the car bottom. Pull the rope end back to the car top to securely fix it. Repeat for remaining ropes. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  |  |  | - Use fall protection equipment if working above the guardrail on a platform ladder or scaffold. |  |  |
|  |  |  |  | - Lift car not to be moved during work. |  |  |
|  |  |  |  | - Keep the top of the car free of excess material, debris, etc |  |  |
| 4 | Access pit | Risk of medical or another emergency involving a Technician descending the ladder or while working in the pit. | 40 | - Alternative 2-way communication used if visual not available e.g. Technician on Top of Car. | 10 | TKE Personnel |
|  |  |  |  | - “Man Down” feature on Smartphone to be used to alert and notify of medical emergencies. |  |  |
|  |  |  |  | - Emergency Services are to be contacted and the Building Manager is advised in the event of a medical emergency and/or if evacuation from the pit is required. |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Before descending the ladder, conduct a visual check of the condition of the floor and ladder and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
|  |  | Risk of Fall Hazard entering the Shaft to descend the ladder to the Pit. | 50 | - Landing doors to be chocked fully open to allow access to the ladder with adjustable door chock/wedge. | 10 | TKE Personnel |
|  |  |  |  | - Confirm the ladder within 1200mm from the edge of the landing door. If not, alternative fall protection may be required. |  |  |
|  |  |  |  | - Lighting in Lift Shaft switched on and working and adequate to sight ladder. |  |  |
|  |  |  |  | - Confirm ladder appears in good working order and is not covered in any liquid/material that might result in a slip. |  |  |
|  |  |  |  | - 3 points of contact to be always maintained while descending or ascending the ladder. Equipment, tools, or materials are not to be carried in hands while on the ladder. All Equipment is to be carried in a backpack. |  |  |
|  |  |  |  | - Other material/equipment not carried in backpack lowered into pit via alternative means (lowering rope). |  |  |
|  |  | Risk of injury from Falling Objects – objects falling from landing onto the Technician in the Pit. | 28 | - No items stored in the vicinity of an edge when doors open. | 14 | TKE Personnel |
|  |  |  |  | - Landing doors closed to a maximum width of 200mm once Technician One has entered the pit. |  |  |
|  |  |  |  | - Materials and tools carried in a backpack or lowered by a Second Technician using a lowering rope. |  |  |
|  |  | Risk of exposure to hazardous/dangerous substances or disease from a build-up of gas discarded needles or sharps, and vermin. | 40 | - If the lift is not a designated Hazardous Goods lift, advise the Building Manager if any suspicious odour or smell is present before entering the shaft pit. | 10 | TKE Personnel |
|  |  |  |  | - Ensure adequate lighting in the shaft and pit and conduct a visual check for evidence of discarded syringes or evidence of vermin in the lift shaft or pit. If identified, report to the Building Manager for removal/treatment. |  |  |
|  |  |  |  | - Report to the site if any evidence of water or inundation before entering the lift pit. |  |  |
| 5 | Install the protection cover of the diversion pulleys. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Conduct a visual check of the condition of the floor and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
| 6 | Egress pit | Risk of medical or another emergency involving a Technician ascending the ladder or while working in the pit. | 40 | - 2-way communication used if visual is not available | 10 | TKE Personnel |
|  |  |  |  | - “Man Down” feature on Smartphone to be used to alert and notify of medical emergencies. |  |  |
|  |  |  |  | - Emergency Services are to be contacted and the Building Manager is advised in the event of a medical emergency and/or if evacuation from the pit is required. |  |  |
|  |  | Risk of Fall Hazard to First Technician using a ladder to exit. | 40 | - Landing doors to be chocked fully open to allow access to the ladder with adjustable door chock/wedge. | 10 | TKE Personnel |
|  |  |  |  | - Confirm the ladder within 1200mm from the edge of the landing door. If not, alternative fall protection may be required. |  |  |
|  |  |  |  | - Lighting in Lift Shaft switched on and working and adequate to sight ladder. |  |  |
|  |  |  |  | - Confirm ladder appears in good working order and is not covered in any liquid/material that might result in a slip. |  |  |
|  |  |  |  | - 3 points of contact to be always maintained while descending or ascending the ladder. Equipment, tools, or materials are not to be carried in hands while on the ladder. All Equipment is to be carried in a backpack. |  |  |
|  |  |  |  | - Other material/equipment not carried in backpack raised out of pit via alternative means (rope). |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Before ascending the ladder, conduct a visual check of the condition of the floor and ladder and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor during servicing. |  |  |
|  |  | Risk of injury from Falling Objects – objects falling from landing onto Technician. | 28 | - No items stored in the vicinity of the edge when doors open. | 14 | TKE Personnel |
| 7 | Move the car to the top of the shaft (rolling back the PSDW rope in the pit at the same time, ensuring no twist in the rope). | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of crush injury from uncontrolled movement of lift car or other crush injury when moving lift car | 50 | - Obtain acknowledgment from other technicians before moving the lift car per SWP 10 (if other personnel are on the lift car). | 10 | TKE Personnel |
|  |  |  |  | - Ensure when moving the car that all materials and occupants remain within the confines of the lift car perimeter. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  |  |  | - Keep the top of the car free of excess material, debris, etc |  |  |
| 8 | Hang a winch from the hook/anchor point (on the traction machine side so the winch rope can be lowered along the counterweight stroke direction without interference) | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  |  |  | - Use fall protection equipment if working above the guardrail on a platform ladder or scaffold. |  |  |
|  |  |  |  | - Lift car not to be moved during work. |  |  |
|  |  |  |  | - Keep the top of the car free of excess material, debris, etc |  |  |
| 9 | Take down one traction rope (installed at the innermost side of the machine) from the car top. Pass through the rope groove of the traction sheave. Then lower it to the bottom of the shaft. | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  |  |  | - Use fall protection equipment if working above the guardrail on a platform ladder or scaffold. |  |  |
|  |  |  |  | - Lift car not to be moved during work. |  |  |
|  |  |  |  | - Keep the top of the car free of excess material, debris, etc |  |  |
| 10 | Pass the traction rope through the counterweight diversion pulley and fix it with the winch rope temporarily. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  |  |  | - Use fall protection equipment if working above the guardrail on a platform ladder or scaffold. |  |  |
|  |  |  |  | - Lift car not to be moved during work. |  |  |
|  |  |  |  | - Keep the top of the car free of excess material, debris, etc |  |  |
| 11 | Operate the winch rope upwards and hoist the traction rope to the top of the shaft. | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  |  |  | - Use fall protection equipment if working above a guardrail on a platform ladder or scaffold |  |  |
|  |  |  |  | - Lift car not to be moved during work. |  |  |
|  |  |  |  | - Keep the top of the car free of excess material, debris, etc |  |  |
| 12 | Install the rope end of the counterweight side and detach the winch rope from the traction rope. | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  |  |  | - Use fall protection equipment if working above the guardrail on a platform ladder or scaffold. |  |  |
|  |  |  |  | - Lift car not to be moved during work. |  |  |
|  |  |  |  | - Keep the top of car free of excess material, debris, etc |  |  |
| 13 | Repeat Steps 9 – 12 to install the remaining traction ropes one by one. | As above |  |  |  | TKE Personnel |
| 14 | Access pit | Risk of medical or another emergency involving a Technician descending the ladder or while working in the pit. | 40 | - Alternative 2-way communication used if visual not available e.g. Technician on Top of Car. | 10 | TKE Personnel |
|  |  |  |  | - “Man Down” feature on Smartphone to be used to alert and notify of medical emergencies. |  |  |
|  |  |  |  | - Emergency Services are to be contacted and the Building Manager is advised in the event of a medical emergency and/or if evacuation from the pit is required. |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Before descending the ladder, conduct a visual check of the condition of the floor and ladder and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
|  |  | Risk of Fall Hazard entering the Shaft to descend the ladder to the Pit. | 50 | - Landing doors to be chocked fully open to allow access to the ladder with adjustable door chock/wedge. | 10 | TKE Personnel |
|  |  |  |  | - Confirm the ladder within 1200mm from the edge of the landing door. If not, alternative fall protection may be required. |  |  |
|  |  |  |  | - Lighting in Lift Shaft switched on and working and adequate to sight ladder. |  |  |
|  |  |  |  | - Confirm ladder appears in good working order and is not covered in any liquid/material that might result in a slip. |  |  |
|  |  |  |  | - 3 points of contact to be always maintained while descending or ascending the ladder. Equipment, tools, or materials are not to be carried in hands while on the ladder. All Equipment is to be carried in a backpack. |  |  |
|  |  |  |  | - Other material/equipment not carried in backpack lowered into pit via alternative means (lowering rope). |  |  |
|  |  | Risk of injury from Falling Objects – objects falling from landing onto the Technician in the Pit. | 28 | - No items stored in the vicinity of the edge when doors open. | 14 | TKE Personnel |
|  |  |  |  | - Landing doors closed to a maximum width of 200mm once Technician One has entered the pit. |  |  |
|  |  |  |  | - Materials and tools carried in a backpack or lowered by a Second Technician using a lowering rope. |  |  |
|  |  | Risk of exposure to hazardous/dangerous substances or disease from a build-up of gas discarded needles or sharps, and vermin. | 40 | - If the lift is not a designated Hazardous Goods lift, advise the Building Manager if any suspicious odour or smell is present before entering the shaft pit. | 10 | TKE Personnel |
|  |  |  |  | - Ensure adequate lighting in the shaft and pit and conduct a visual check for evidence of discarded syringes or evidence of vermin in the lift shaft or pit. If identified, report to the Building Manager for removal/treatment. |  |  |
|  |  |  |  | - Report to the site if any evidence of water or inundation before entering the lift pit. |  |  |
| 15 | With secondary protection on the counterweight frame, adjust the counterweight frame to a proper distance from the buffer and securely fix it. | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Conduct a visual check of the condition of the floor and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 10 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |
| 16 | Egress pit | Risk of medical or another emergency involving a Technician ascending the ladder or while working in the pit. | 40 | - 2-way communication used if visual is not available | 10 | TKE Personnel |
|  |  |  |  | - “Man Down” feature on Smartphone to be used to alert and notify of medical emergencies. |  |  |
|  |  |  |  | - Emergency Services are to be contacted and the Building Manager is advised in the event of a medical emergency and/or if evacuation from the pit is required. |  |  |
|  |  | Risk of Fall Hazard to First Technician using a ladder to exit. | 40 | - Landing doors to be chocked fully open to allow access to the ladder with adjustable door chock/wedge. | 10 | TKE Personnel |
|  |  |  |  | - Confirm the ladder within 1200mm from the edge of the landing door. If not, alternative fall protection may be required. |  |  |
|  |  |  |  | - Lighting in Lift Shaft switched on and working and adequate to sight ladder. |  |  |
|  |  |  |  | - Confirm ladder appears in good working order and is not covered in any liquid/material that might result in a slip. |  |  |
|  |  |  |  | - 3 points of contact to be always maintained while descending or ascending the ladder. Equipment, tools, or materials are not to be carried in hands while on the ladder. All Equipment is to be carried in a backpack. |  |  |
|  |  |  |  | - Other material/equipment not carried in backpack raised out of pit via alternative means (rope). |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Before ascending the ladder, conduct a visual check of the condition of the floor and ladder and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor during servicing. |  |  |
|  |  | Risk of injury from Falling Objects – objects falling from landing onto Technician. | 28 | - No items stored in the vicinity of the edge when doors open. | 14 | TKE Personnel |

| # Activity 16 - Counterweight Fillers Top-Up |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Job Step Break the job down into logical steps |  | Potential Hazards Identify the hazards associated with each step. Examine each to find possibilities that could lead to an accident. | Initial Risk Level | Controls List the actions necessary to eliminate or minimise the hazards that could lead to an accident, injury, or occupational illness. | Final Risk Level | Person Responsible Identify the Persons responsible for ensuring controls are implemented. |
| 1 | Access pit | Risk of medical or another emergency involving a Technician descending the ladder or while working in the pit. | 40 | - Alternative 2-way communication used if visual not available e.g. Technician on Top of Car. | 10 | TKE Personnel |
|  |  |  |  | - “Man Down” feature on Smartphone to be used to alert and notify of medical emergencies. |  |  |
|  |  |  |  | - Emergency Services are to be contacted and the Building Manager is advised in the event of a medical emergency and/or if evacuation from the pit is required. |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Before descending the ladder, conduct a visual check of the condition of the floor and ladder and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
|  |  | Risk of Fall Hazard entering the Shaft to descend a ladder to the Pit. | 50 | - Landing doors to be chocked fully open to allow access to the ladder with adjustable door chock/wedge. | 10 | TKE Personnel |
|  |  |  |  | - Confirm ladder is within 1200mm from the edge of the landing door. If not, alternative fall protection may be required. |  |  |
|  |  |  |  | - Lighting in Lift Shaft switched on and working and adequate to sight ladder. |  |  |
|  |  |  |  | - Confirm ladder appears in good working order and is not covered in any liquid/material that might result in a slip. |  |  |
|  |  |  |  | - 3 points of contact to be always maintained while descending or ascending the ladder. Equipment, tools, or materials are not to be carried in hands while on the ladder. All Equipment is to be carried in a backpack. |  |  |
|  |  |  |  | - Other material/equipment not carried in backpack lowered into pit via alternative means (lowering rope). |  |  |
|  |  | Risk of injury from Falling Objects – objects falling from landing onto the Technician in the Pit. | 28 | - No items stored in the vicinity of the edge when doors open. | 14 | TKE Personnel |
|  |  |  |  | - Landing doors closed to a maximum width of 200mm once Technician One has entered the pit. |  |  |
|  |  |  |  | - Materials and tools carried in a backpack or lowered by the Second Technician using a lowering rope. |  |  |
|  |  | Risk of exposure to hazardous/dangerous substances or disease from build-up of gas discarded needles or sharps, and vermin. | 40 | - If the lift is not a designated Hazardous Goods lift, advise the Building Manager if any suspicious odour or smell is present before entering the shaft pit. | 10 | TKE Personnel |
|  |  |  |  | - Ensure adequate lighting in the shaft and pit and conduct a visual check for evidence of discarded syringes or evidence of vermin in the lift shaft or pit. If identified, report to the Building Manager for removal/treatment. |  |  |
|  |  |  |  | - Report to the site if any evidence of water or inundation before entering the lift pit. |  |  |
| 2 | From the bottom landing, remove the secondary protection device | Risk of crushing due to improper rigging or failure of lifting point | 56 | - Lifting points to have pull testing conducted. | 20 | TKE Personnel |
|  |  |  |  | - All rigging works to be carried out by a licensed and competent rigger. |  |  |
|  |  |  |  | - All lifting equipment checked before use and tested within the last 12 months. |  |  |
|  |  |  |  | - Use rigging practices and correct technique to lift part. |  |  |
|  |  |  |  | - No person directly under the load while the load is moving / in the air. |  |  |
|  |  |  |  | - Observe SWL of lifting equipment |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Conduct a visual check of the condition of the floor and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
| 3 | Add counterweight fillers as required to balance the weight of the car. Fix firmly in place. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Conduct a visual check of the condition of the floor and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
| 4 | Install guarding on top and front side of the counterweight | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Conduct a visual check of the condition of the floor and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 10 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |
| 5 | Egress pit | Risk of medical or another emergency involving a Technician ascending the ladder or while working in the pit. | 40 | - 2-way communication used if visual is not available | 10 | TKE Personnel |
|  |  |  |  | - “Man Down” feature on Smartphone to be used to alert and notify of medical emergencies. |  |  |
|  |  |  |  | - Emergency Services are to be contacted and the Building Manager is advised in the event of a medical emergency and/or if evacuation from the pit is required. |  |  |
|  |  | Risk of Fall Hazard to First Technician using a ladder to exit. | 40 | - Landing doors to be chocked fully open to allow access to the ladder with adjustable door chock/wedge. | 10 | TKE Personnel |
|  |  |  |  | - Confirm the ladder within 1200mm from the edge of the landing door. If not, alternative fall protection may be required. |  |  |
|  |  |  |  | - Lighting in Lift Shaft switched on and working and adequate to sight ladder. |  |  |
|  |  |  |  | - Confirm ladder appears in good working order and is not covered in any liquid/material that might result in a slip. |  |  |
|  |  |  |  | - 3 points of contact to be always maintained while descending or ascending the ladder. Equipment, tools, or materials are not to be carried in hands while on the ladder. All Equipment is to be carried in a backpack. |  |  |
|  |  |  |  | - Other material/equipment not carried in backpack raised out of pit via alternative means (rope). |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Before ascending the ladder, conduct a visual check of the condition of the floor and ladder and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor during servicing. |  |  |
|  |  | Risk of injury from Falling Objects – objects falling from landing onto Technician. | 28 | - No items stored in the vicinity of an edge when doors open. | 14 | TKE Personnel |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Before ascending the ladder, conduct a visual check of the condition of the floor and ladder and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor during servicing. |  |  |

| # Activity 17 - Overspeed Governor Roping |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Job Step Break the job down into logical steps |  | Potential Hazards Identify the hazards associated with each step. Examine each to find possibilities that could lead to an accident. | Initial Risk Level | Controls List the actions necessary to eliminate or minimise the hazards that could lead to an accident, injury, or occupational illness. | Final Risk Level | Person Responsible Identify the Persons responsible for ensuring controls are implemented. |
| 1 | From the top of the car, lower the car until the traction machine steel wire ropes are under stress. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
| 2 | Place the governor rope reel on top of the car. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
| 3 | Another team to access the pit | Risk of medical or other emergency involving a Technician descending the ladder or while working in the pit. | 40 | - Alternative 2-way communication used if visual not available e.g. Technician on Top of Car. | 10 | TKE Personnel |
|  |  |  |  | - “Man Down” feature on Smartphone to be used to alert and notify of medical emergencies. |  |  |
|  |  |  |  | - Emergency Services are to be contacted and the Building Manager is advised in the event of a medical emergency and/or if evacuation from the pit is required. |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Before descending the ladder, conduct a visual check of the condition of the floor and ladder and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
|  |  | Risk of Fall Hazard entering the Shaft to descend the ladder to the Pit. | 50 | - Landing doors to be chocked fully open to allow access to the ladder with adjustable door chock/wedge. | 10 | TKE Personnel |
|  |  |  |  | - Confirm the ladder within 1200mm from the edge of the landing door. If not, alternative fall protection may be required. |  |  |
|  |  |  |  | - Lighting in Lift Shaft switched on and working and adequate to sight ladder. |  |  |
|  |  |  |  | - Confirm ladder appears in good working order and is not covered in any liquid/material that might result in a slip. |  |  |
|  |  |  |  | - 3 points of contact to be maintained at all times while descending or ascending the ladder. Equipment, tools, or materials are not to be carried in hands while on the ladder. All Equipment is to be carried in a backpack. |  |  |
|  |  |  |  | - Other material/equipment not carried in backpack lowered into pit via alternative means (lowering rope). |  |  |
|  |  | Risk of injury from Falling Objects – objects falling from landing onto the Technician in the Pit. | 28 | - No items stored in the vicinity of an edge when doors open. | 14 | TKE Personnel |
|  |  |  |  | - Landing doors closed to a maximum width of 200mm once Technician One has entered the pit. |  |  |
|  |  |  |  | - Materials and tools carried in a backpack or lowered by the Second Technician using a lowering rope. |  |  |
|  |  | Risk of exposure to hazardous/dangerous substances or disease from a build-up of gas discarded needles or sharps, and vermin. | 40 | - If the lift is not a designated Hazardous Goods lift, advise the Building Manager if any suspicious odour or smell is present before entering the shaft pit. | 10 | TKE Personnel |
|  |  |  |  | - Ensure adequate lighting in the shaft and pit and conduct a visual check for evidence of discarded syringes or evidence of vermin in the lift shaft or pit. If identified, report to the Building Manager for removal/treatment. |  |  |
|  |  |  |  | - Report to the site if any evidence of water or inundation before entering the lift pit. |  |  |
| 4 | Rope the overspeed governor (from the top of the car and in the pit – rope to come to the side of the car) Cont. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Conduct a visual check of the condition of the floor and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
| 5 | Fix the ropes and secure them with relevant bolts. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Conduct a visual check of the condition of the floor and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
| 6 | Check whether the overspeed governor and safety gear mechanism are functional. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Conduct a visual check of the condition of the floor and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor. |  |  |
| 7 | Egress pit | Risk of medical or another emergency involving a Technician ascending the ladder or while working in the pit. | 40 | - 2-way communication used if visual is not available | 10 | TKE Personnel |
|  |  |  |  | - “Man Down” feature on Smartphone to be used to alert and notify of medical emergencies. |  |  |
|  |  |  |  | - Emergency Services are to be contacted and the Building Manager is advised in the event of a medical emergency and/or if evacuation from the pit is required. |  |  |
|  |  | Risk of Fall Hazard to First Technician using a ladder to exit. | 40 | - Landing doors to be chocked fully open to allow access to a ladder with adjustable door chock/wedge. | 10 | TKE Personnel |
|  |  |  |  | - Confirm the ladder within 1200mm from the edge of the landing door. If not, alternative fall protection may be required. |  |  |
|  |  |  |  | - Lighting in Lift Shaft switched on and working and adequate to sight ladder. |  |  |
|  |  |  |  | - Confirm ladder appears in good working order and is not covered in any liquid/material that might result in a slip. |  |  |
|  |  |  |  | - 3 points of contact to be always maintained while descending or ascending the ladder. Equipment, tools, or materials are not to be carried in hands while on a ladder. All Equipment is to be carried in a backpack. |  |  |
|  |  |  |  | - Other material/equipment not carried in backpack raised out of pit via alternative means (rope). |  |  |
|  |  | Risk of Slip / Trip and Fall in Pit. | 14 | - Check adequate lighting in the shaft and pit. | 6 | TKE Personnel |
|  |  |  |  | - Before ascending the ladder, conduct a visual check of the condition of the floor and ladder and check for oil and water. |  |  |
|  |  |  |  | - Regular cleaning of the floor during servicing. |  |  |
|  |  | Risk of injury from Falling Objects – objects falling from landing onto Technician. | 28 | - No items stored in the vicinity of the edge when doors open. | 14 | TKE Personnel |
| 8 | Conduct inspection speed commissioning. Lower the car until the PSDW and PSDW rope hook are loosened and slack. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |

| # Activity 18 - Inspection Running Commissioning |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Job Step Break the job down into logical steps |  | Potential Hazards Identify the hazards associated with each step. Examine each to find possibilities that could lead to an accident. | Initial Risk Level | Controls List the actions necessary to eliminate or minimise the hazards that could lead to an accident, injury, or occupational illness. | Final Risk Level | Person Responsible Identify the Persons responsible for ensuring controls are implemented. |
| 1 | Inspect installation for readiness for inspection mode operation. Make necessary adjustments as required | Risk of fall from height from top of car into shaft/pit | 50 | - Keep within the perimeter of the lift car whilst traveling. | 10 | Tuner / Commissioner |
|  |  |  |  | - Ensure landing doors are closed and locked traveling from the landing. |  |  |
|  |  |  |  | - Ensure the top of the lift car is level with the landing before entry. |  |  |
| 2 | Inspect installation for readiness for inspection mode operation. Make any necessary adjustments as required | Risk of falling objects | 40 | - No work to be completed in the lift shaft or the pit. | 10 | Tuner / Commissioner |
|  |  |  |  | - Keep the top of the car and shaft free of excess material, debris, etc |  |  |
|  |  | Risk of electric shock | 50 | - Mains power supply to be terminated into lift circuit breaker panel by the site electrical subcontractor or qualified electrician. Lock and tag out procedure to be completed. | 10 | Tuner / Commissioner |
|  |  |  |  | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. |  |  |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |
|  |  | Risk of slip/trip | 40 | - Keep the top of the car and shaft free of excess material, debris, etc | 10 | Tuner / Commissioner |
|  |  | Unauthorized access to the lift circuit breaker/controller | 56 | - Ensure the lift controller door is locked before departing the work area. | 20 | Tuner / Commissioner |
| 3 | Inspect/check all wiring connections in the control panel. | Risk of electric shock | 50 | - Ensure the circuit breaker has a lockout / tag-out facility and is used. | 20 | Tuner / Commissioner |
|  |  |  |  | - Confirm zero energy state with multi-meter. |  |  |
|  |  | Unauthorized access to the lift circuit breaker/controller | 56 | - Ensure the lift controller door is locked before departing the work area. | 20 | Tuner / Commissioner |
| 4 | Ascertain continuity (no shorts or earth faults) in the safety circuits, controller, car top circuits, and shaft loom. | Risk of fall from height from top of car into shaft/pit | 50 | - Keep within the perimeter of the lift car whilst traveling. | 10 | Tuner / Commissioner |
|  |  |  |  | - Ensure landing doors are closed and locked traveling from the landing. |  |  |
|  |  |  |  | - Ensure the top of the lift car is level with the lobby landing before entry. |  |  |
|  |  | Risk of falling objects | 40 | - No work to be completed in the lift shaft or the pit. | 10 | Tuner / Commissioner |
|  |  |  |  | - Keep the top of the car and shaft free of excess material, debris, etc |  |  |
|  |  | Risk of electric shock | 50 | - Ensure the circuit breaker has a lockout / tag-out facility and is used. | 20 | Tuner / Commissioner |
|  |  |  |  | - Confirm zero energy state with multi-meter. |  |  |
|  |  | Risk of slip/trip | 40 | - Keep the top of the car and shaft free of excess material, debris, etc | 10 | Tuner / Commissioner |
|  |  | Unauthorized access to the lift circuit breaker/controller | 56 | - Ensure the lift controller door is locked before departing the work area. | 20 | Tuner / Commissioner |
| 5 | Energize the lift circuit breaker and verify the correct “fire up” sequence of the lift controller. | Risk of electric shock | 50 | - Follow the lock and tag out procedure if departing the area and the lift is not in a safe state. | 20 | Tuner / Commissioner |
|  |  |  |  | - All commissioning and testing work to be performed by qualified technicians only. |  |  |
|  |  |  |  | - Circuit breaker must be turned off before any wiring works in the controller are completed. |  |  |
|  |  | Unauthorized access to the lift circuit breaker/controller | 56 | Ensure the lift controller door is locked before departing the work area. | 20 | Tuner / Commissioner |
| 6 | Complete lift motor functional tests and checks. | Risk of electric shock | 50 | - Follow the lock and tag out procedure if departing the area and the lift is not in a safe state. | 20 | Tuner / Commissioner |
|  |  |  |  | - All commissioning and testing work to be performed by qualified technicians only. |  |  |
|  |  |  |  | - Circuit breaker must be turned off before any wiring works in the controller are completed. |  |  |
|  |  | Unauthorized access to the lift circuit breaker/controller | 56 | Ensure the lift controller door is locked before departing the work area. | 20 | Tuner / Commissioner |
|  |  | Risk of fall from height | 40 | Ensure the lift car is level with the landing when inspecting/verifying lift operation in the shaft. | 10 | Tuner / Commissioner |
|  |  | Risk of crushing | 56 | No personnel in the lift shaft whilst testing is in progress. | 20 | Tuner / Commissioner |
| 7 | Test and confirm lift car top inspection controls functional | Risk of electric shock | 50 | - Follow the lock and tag out procedure if departing the area and the lift is not in a safe state. | 20 | Tuner / Commissioner |
|  |  |  |  | - All commissioning and testing work to be performed by qualified technicians only. |  |  |
|  |  |  |  | - Circuit breaker must be turned off before any wiring works in the controller are completed. |  |  |
|  |  | Risk of fall from height | 40 | - Keep within the perimeter of the lift car handrail. | 10 | Tuner / Commissioner |
|  |  |  |  | - Ensure landing doors are closed and locked traveling from the landing. |  |  |
|  |  | Risk of falling objects | 40 | - No personnel are to be in the lift shaft when it is moved. | 10 | Tuner / Commissioner |
|  |  |  |  | - No personnel to work above or below the lift car. |  |  |
|  |  |  |  | - Keep lift car top and shaft free of excess material, debris, etc |  |  |
|  |  | Risk of crushing | 56 | - Keep within the perimeter of the lift car handrails whilst traveling. | 20 | Tuner / Commissioner |
|  |  | Risk of slip/trip | 40 | - Keep lift car top free of excess material, debris, etc | 10 | Tuner / Commissioner |
|  |  |  |  | - Always stand on the lift car roof and not the car superstructure. |  |  |
|  |  | Unauthorised access to car top controls | 56 | - Ensure landing doors are closed and locked when leaving the area. | 20 | Tuner / Commissioner |

| # Activity 19 - Hoisting Assembly Dismantlement |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Job Step Break the job down into logical steps |  | Potential Hazards Identify the hazards associated with each step. Examine each to find possibilities that could lead to an accident. | Initial Risk Level | Controls List the actions necessary to eliminate or minimise the hazards that could lead to an accident, injury, or occupational illness. | Final Risk Level | Person Responsible Identify the Persons responsible for ensuring controls are implemented. |
| 1 | From the top landing, place the rope reel on the landing and access the top of the car. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
| 2 | Remove the PSDW rope hook from the connecting base and roll back onto the reel on landing by operating the PSDW. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
| 3 | Remove the PSDW | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
| 4 | Move the car downwards on “Inspection” mode and remove the blocstop, connecting bases, and remaining car top protection equipment. Place guardrails back in place if removed earlier on (and the car top protection guardrails were installed). | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 10 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |
| 5 | Move the car upward on “Inspection” mode and at the same time roll the blocstop rope back to the rope reel inside the lift car. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
| 6 | At the top of the shaft, remove the hook of the blocstop rope and the guide pulley of the PSDW rope from the shaft ceiling (ensuring the hook/anchor point is tagged) | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 6 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of falling from heights | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  |  |  | - Use fall protection equipment if working above the guardrail on a platform ladder or scaffold. |  |  |
|  |  |  |  | - Lift car not to be moved during works |  |  |
|  |  |  |  | - Keep the top of car free of excess material, debris, etc |  |  |

| # Activity 20 - Other Elevator Components Installation (Install Car Electricals) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Job Step Break the job down into logical steps |  | Potential Hazards Identify the hazards associated with each step. Examine each to find possibilities that could lead to an accident. | Initial Risk Level | Controls List the actions necessary to eliminate or minimise the hazards that could lead to an accident, injury, or occupational illness. | Final Risk Level | Person Responsible Identify the Persons responsible for ensuring controls are implemented. |
| 1 | Access the top of the car | Risk of Fall from Heights from Top of Car. | 50 | - Confirm lift is at a level where the top of the car is in line with the floor level where access is being made. | 6 | TKE Personnel |
|  |  |  |  | - Handrails in place on top of the car roof where the gap between the shaft wall and the outside perimeter of the roof exceeds 300mm. |  |  |
|  |  |  |  | - If none are in place, alternative fall protection (restraint harness) or other control is to be used (lift lowered to lowest terminal position). |  |  |
|  |  |  |  | - Harnesses should not be used when the car is moving due to the possibility of entanglement. |  |  |
|  |  |  |  | - Car should be secured and stopped from moving before use. |  |  |
|  |  | Risk of injury to other TKE Personnel in a pit or damage to lift equipment from objects/tools falling from the top of the lift car. | 40 | - Where possible ensure work on top of the car is conducted with the lift in the lowest possible position or no work in the pit while other work taking place on top of the car. | 10 | TKE Personnel |
|  |  |  |  | - Toe guard in place around the top of the car. |  |  |
|  |  |  |  | - Ensure hand tools and small equipment are placed in a bag and not loose on top of the car roof. All equipment is stored in the middle of the car roof and not on edge. Remove from pockets items that can potentially fall. |  |  |
|  |  |  |  | - Do not overcrowd the top of the car with too many people or equipment. Leave enough space so personnel can safely move around the top of the car. |  |  |
|  |  | Risk of trip and fall or twisting injury to knee and ankle when getting on top of the lift car. | 40 | - With doors open, turn on the shaft light. Ensure there is adequate lighting to get on and off top car safely. | 10 | TKE Personnel |
|  |  |  |  | - Ensure the lift is in a fully lowered position before climbing up or down onto the top of the lift car. No jumping from the top of the car down to landing (per TKE SWP 16). |  |  |
|  |  |  |  | - Ensure the floor landing and top of the car are clear of tools or equipment to ensure a safe area to step when getting on or off the lift car. |  |  |
| 2 | Set out troughing and conduit work on the car roof | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 7 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 20 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |
|  |  |  |  | - Ensure the lift Main Circuit Breaker is locked and tagged as per SWP18. |  |  |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  | Risk of fall from height | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
| 3 | Install and terminate the top of car wiring, car fan, top of car light, and inspection box controls. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 7 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of electric shock | 50 | - Ensure power tools, electrical leads, winches, and power boxes have been tested and tagged by a licensed electrician. | 20 | TKE Personnel |
|  |  |  |  | - Keep electrical supply cables clear of shaft components and floor, and all electrical leads suspended by approved lead stands/hooks 2 meters above the ground. |  |  |
|  |  | Risk of fall from height | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  |  |  | - Maintain three points of contact when using a ladder. |  |  |
|  |  |  |  | - Platform ladder to be used. |  |  |
|  |  | Risk of falling objects | 40 | - No personnel to work above or below the car. | 10 | TKE Personnel |
|  |  |  |  | - Keep car top free of excess material, debris, etc |  |  |
| 4 | Install and wire lift car operating panels, GPO, and emergency lighting in the lift car. | Risk of fall from height | 40 | - Always keep within the perimeter of the lift car top handrail. | 10 | TKE Personnel |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | TKE Personnel |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 7 | TKE Personnel |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
| 5 | Egress top of the car | Risk of crush / traumatic injury to Technician through the uncontrolled movement of the Lift when getting off the top of the car. | 50 | - Confirm lift is at a level where the top of the car is in line with the floor level where egress is being made per TKE SWP 16). | 10 | TKE Personnel |
|  |  |  |  | - Landing doors to be chocked fully open to allow egress to the top of the car with adjustable door chock/wedge. |  |  |
|  |  | Risk of trip and fall or twisting injury to knee and ankle when getting off the lift car. | 28 | - With doors open, turn on the shaft light. Ensure there is adequate lighting to get on and off top car safely. | 14 | TKE Personnel |
|  |  |  |  | - Ensure the lift is in a fully lowered position before climbing up or down onto the top of lift car. No jumping from the top of car down to landing (per TKE SWP 16). |  |  |
|  |  |  |  | - Ensure the floor landing and top of the car is clear of tools or equipment to ensure a safe area to step when getting on or off the lift car. |  |  |
|  |  | Fall from Height – the risk of fall from the top of Lift Car down shaft or between shaft wall and outside perimeter of the car top roof. | 50 | - Confirm lift is at a level where the top of car is in line with the floor level where egress is being made from. | 10 | TKE Personnel |
|  |  |  |  | - Handrails in place on top of car roof where the gap between shaft wall and outside perimeter of the roof exceeds 300mm. |  |  |
|  |  |  |  | - If none is in place, alternative fall protection (restraint harness) or other control is to be used (lift lowered to lowest terminal position). Harnesses should not be used when car is moving due to possibility of entanglement. Car should be secured and stopped from moving before use. |  |  |

| # Activity 21 - Normal Speed Running Commissioning |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Job Step Break the job down into logical steps |  | Potential Hazards Identify the hazards associated with each step. Examine each to find possibilities that could lead to an accident. | Initial Risk Level | Controls List the actions necessary to eliminate or minimise the hazards that could lead to an accident, injury, or occupational illness. | Final Risk Level | Person Responsible Identify the Persons responsible for ensuring controls are implemented. |
| 1 | Initial risk assessment | Risk of electric shock | 50 | - All testing and commissioning are conducted by a licensed Electrician. | 10 | Tuner / Commissioner |
|  |  |  |  | - Confirm no personnel or passengers using the Lift. |  |  |
|  |  |  |  | - Confirm no bridging leads on the Controller. Review Bridging Lead register. No bridging leads to be left in place that compromise safety during testing. |  |  |
|  |  |  |  | - Before inspection (if not already energized), conduct a de-energised visual inspection. Remove tests and tags. |  |  |
|  |  |  |  | - Ensure the area around CB or Controller is clear of trip hazards and conductive material such as water on the floor before de-energisation. |  |  |
|  |  |  |  | - Conduct a risk assessment. Refer to SWP 46. Wear PPE such as riggers gloves, and safety glasses as minimum. Check that the incoming supply is enclosed, and switches are insulated (in place if a new Controller is installed). |  |  |
|  |  |  |  | - If not, then use additional PPE including LV gloves and insulated mat. |  |  |
|  |  |  |  | - Have LVR rescue and a second technician available as a spotter if identified as part of risk assessment in line with Electrical COP. |  |  |
|  |  |  |  | - Run Lift on “Inspection” mode. |  |  |
| 2 | Check and meter all circuits, earthing, Insulation and safety circuits | Risk of electric shock | 50 | - All testing and commissioning are conducted by a licensed Electrician. | 10 | Tuner / Commissioner |
|  |  |  |  | - Ensure the area around CB or Controller is clear of trip hazards and conductive material such as water on the floor before de-energisation. |  |  |
|  |  |  |  | - Wear PPE such as riggers gloves, and safety glasses as minimum. Check that the incoming supply is enclosed, and switches are insulated (in place if a new Controller is installed). |  |  |
|  |  |  |  | - If not, then use additional PPE including LV gloves and insulated mat. |  |  |
|  |  |  |  | - Have LVR rescue and a second technician available as a spotter if identified as part of risk assessment in line with Electrical COP. |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | Tuner / Commissioner |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 7 | Tuner / Commissioner |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
| 3 | Check all mechanical equipment. | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | Tuner / Commissioner |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 7 | Tuner / Commissioner |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of crush injury from moving/operating parts. | 40 | - Ensure all guards are in place to protect against moving parts. | 7 | Tuner / Commissioner |
|  |  |  |  | - Wear PPE including eye protection and footwear. |  |  |
|  |  |  |  | - Do not touch moving parts, use hand tools. |  |  |
|  |  |  |  | - Listen for noises and check clearances of moving parts. |  |  |
| 4 | Remove tags and locks operate the main switch and auxiliary switches test for power and test RCDs | Risk of electric shock | 50 | - All testing and commissioning are conducted by a licensed Electrician. | 10 | Tuner / Commissioner |
|  |  |  |  | - Ensure the area around CB or Controller is clear of trip hazards and conductive material such as water on the floor before de-energisation. |  |  |
|  |  |  |  | - Wear PPE such as riggers gloves, and safety glasses as minimum. Check that the incoming supply is enclosed and switches are insulated (in place if a new Controller is installed). |  |  |
|  |  |  |  | - If not, then use additional PPE including LV gloves and insulated mat. |  |  |
|  |  |  |  | - Have LVR rescue and a second technician available as a spotter if identified as part of risk assessment in line with Electrical COP. |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | Tuner / Commissioner |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 7 | Tuner / Commissioner |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
| 5 | Run Car on inspection mode | Risk of crush injury from moving/operating parts. | 40 | - Ensure all guards are in place to protect against moving parts. | 7 | Tuner / Commissioner |
|  |  |  |  | - Wear PPE including eye protection and footwear. |  |  |
|  |  |  |  | - Do not touch moving parts, use hand tools. |  |  |
|  |  |  |  | - Listen for noises and check clearances of moving parts. |  |  |
| 6 | Drive the car to the top floor enter the car top place it on inspection mode and operate the stop button. | Risk of fall from height from top of car into shaft/pit | 50 | - Handrails in place on top of the car where the gap between the shaft wall and outside perimeter exceeds 300mm. If none is in place, alternative fall protection is to be used. | 10 | Tuner / Commissioner |
|  |  |  |  | - Harnesses should not be used when the lift car is moving due to the possibility of entanglement. The lift car should be secured and stopped from moving before use. |  |  |
|  |  |  |  | - Maintain all body within the limits of the lift car top whilst traveling. |  |  |
|  |  |  |  | - Ensure kickboards and handrails correctly assembled. |  |  |
|  |  | Risk of crush injury from moving/operating parts. | 40 | - Ensure all guards are in place to protect against moving parts. | 7 | Tuner / Commissioner |
|  |  |  |  | - Wear PPE including eye protection and footwear. |  |  |
|  |  |  |  | - Do not touch moving parts, use hand tools. |  |  |
|  |  |  |  | - Listen for noises and check clearances of moving parts. |  |  |
| 7 | Proceed to the bottom floor enter the pit and check all mechanical and electrical equipment | Risk of electric shock | 50 | - All testing and commissioning conducted by licenced Electrician. | 10 | Tuner / Commissioner |
|  |  |  |  | - Ensure area around CB or Controller is clear of trip hazards and conductive material such as water on floor prior to de-energisation. |  |  |
|  |  |  |  | - Wear PPE such as riggers gloves, safety glasses as minimum. Check that incoming supply enclosed and switches are insulated (in place if new Controller installed). |  |  |
|  |  |  |  | - If not, then use additional PPE including LV gloves and insulated mat. |  |  |
|  |  |  |  | - Have LVR rescue and second technician available as spotter if identified as part of risk assessment in line with Electrical COP. |  |  |
|  |  | Risk of fall from height from top of car into shaft/pit | 50 | - Handrails in place on top of car where the gap between shaft wall and outside perimeter exceeds 300mm. If none in place, alternative fall protection to be used. | 10 | Tuner / Commissioner |
|  |  |  |  | - Harnesses should not be used when the lift car is moving due to the possibility of entanglement. The lift car should be secured and stopped from moving before use. |  |  |
|  |  |  |  | - Maintain all body within the limits of the lift car top whilst traveling. |  |  |
|  |  |  |  | - Ensure kickboards and handrails are correctly assembled. |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | Tuner / Commissioner |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 7 | Tuner / Commissioner |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
| 8 | Enter the top of car and drive the car up so that a 2nd commissioner can check and adjust the operation of the car doors and pin | Risk of crush injury from moving/operating parts. | 40 | - Ensure all guards are in place to protect against moving parts. | 7 | Tuner / Commissioner |
|  |  |  |  | - Wear PPE including eye protection and footwear. |  |  |
|  |  |  |  | - Do not touch moving parts, use hand tools. |  |  |
|  |  |  |  | - Listen for noises and check clearances of moving parts. |  |  |
|  |  | Risk of electric shock | 50 | - All testing and commissioning are conducted by a licenced Electrician. | 10 | Tuner / Commissioner |
|  |  |  |  | - Ensure the area around CB or Controller is clear of trip hazards and conductive material such as water on the floor before de-energisation. |  |  |
|  |  |  |  | - Wear PPE such as riggers gloves, and safety glasses as minimum. Check that the incoming supply is enclosed and switches are insulated (in place if a new Controller is installed). |  |  |
|  |  |  |  | - If not, then use additional PPE including LV gloves and insulated mat. |  |  |
|  |  |  |  | - Have LVR rescue and a second technician available as a spotter if identified as part of risk assessment in line with Electrical COP. |  |  |
|  |  | Risk of fall from height from top of car into shaft/pit | 50 | - Handrails in place on top of the car where the gap between the shaft wall and outside perimeter exceeds 300mm. If none is in place, alternative fall protection is to be used. | 10 | Tuner / Commissioner |
|  |  |  |  | - Harnesses should not be used when the lift car is moving due to the possibility of entanglement. The lift car should be secured and stopped from moving before use. |  |  |
|  |  |  |  | - Maintain all body within the limits of the lift car top whilst traveling. |  |  |
|  |  |  |  | - Ensure kickboards and handrails are correctly assembled. |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | Tuner / Commissioner |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
| 9 | Drive car top-level landing doors adjust and pin | Risk of electric shock | 50 | - All testing and commissioning are conducted by a licensed Electrician. | 10 | Tuner / Commissioner |
|  |  |  |  | - Ensure the area around CB or Controller is clear of trip hazards and conductive material such as water on the floor before de-energisation. |  |  |
|  |  |  |  | - Wear PPE such as riggers gloves, and safety glasses as minimum. Check that the incoming supply is enclosed and switches are insulated (in place if a new Controller is installed). |  |  |
|  |  |  |  | - If not, then use additional PPE including LV gloves and insulated mat. |  |  |
|  |  |  |  | - Have LVR rescue and a second technician available as a spotter if identified as part of risk assessment in line with Electrical COP. |  |  |
|  |  | Risk of fall from height from top of car into shaft/pit | 50 | - Handrails in place on top of the car where the gap between the shaft wall and outside perimeter exceeds 300mm. | 10 | Tuner / Commissioner |
|  |  |  |  | - If none is in place, alternative fall protection is to be used. Harnesses should not be used when the lift car is moving due to the possibility of entanglement. The lift car should be secured and stopped from moving before use. |  |  |
|  |  |  |  | - Maintain all body within the limits of the lift car top whilst traveling. |  |  |
|  |  |  |  | - Ensure kickboards and handrails are correctly assembled. |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | Tuner / Commissioner |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
| 10 | Adjust all landing doors and pin | Risk of electric shock | 50 | - All testing and commissioning are conducted by a licensed Electrician. | 10 | Tuner / Commissioner |
|  |  |  |  | - Ensure the area around CB or Controller is clear of trip hazards and conductive material such as water on the floor before de-energisation. |  |  |
|  |  |  |  | - Wear PPE such as riggers gloves, and safety glasses as minimum. Check that the incoming supply is enclosed and switches are insulated (in place if a new Controller is installed). |  |  |
|  |  |  |  | - If not, then use additional PPE including LV gloves and insulated mat. |  |  |
|  |  |  |  | - Have LVR rescue and a second technician available as a spotter if identified as part of risk assessment in line with Electrical COP. |  |  |
|  |  | Risk of fall from height from top of car into shaft/pit | 50 | - Handrails in place on top of the car where the gap between the shaft wall and outside perimeter exceeds 300mm. If none is in place, alternative fall protection is to be used. | 10 | Tuner / Commissioner |
|  |  |  |  | - Harnesses should not be used when the lift car is moving due to the possibility of entanglement. The lift car should be secured and stopped from moving before use. |  |  |
|  |  |  |  | - Maintain all body within the limits of the lift car top whilst traveling. |  |  |
|  |  |  |  | - Ensure kickboards and handrails are correctly assembled. |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | Tuner / Commissioner |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
| 11 | Adjust all limit switches magnets and pin | Risk of electric shock | 50 | - All testing and commissioning are conducted by a licensed Electrician. | 10 | Tuner / Commissioner |
|  |  |  |  | - Ensure the area around CB or Controller is clear of trip hazards and conductive material such as water on the floor before de-energisation. |  |  |
|  |  |  |  | - Wear PPE such as riggers gloves, and safety glasses as minimum. Check that the incoming supply is enclosed and switches are insulated (in place if the new Controller installed). |  |  |
|  |  |  |  | - If not, then use additional PPE including LV gloves and insulated mat. |  |  |
|  |  |  |  | - Have LVR rescue and a second technician available as a spotter if identified as part of risk assessment in line with Electrical COP. |  |  |
|  |  | Risk of fall from height from top of car into shaft/pit | 50 | - Handrails in place on top of the car where the gap between the shaft wall and outside perimeter exceeds 300mm. If none is in place, alternative fall protection is to be used. Harnesses should not be used when the lift car is moving due to the possibility of entanglement. The lift car should be secured and stopped from moving before use. | 10 | Tuner / Commissioner |
|  |  |  |  | - Maintain all body within the limits of the lift car top whilst traveling. |  |  |
|  |  |  |  | - Ensure kickboards and handrails are correctly assembled. |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | Tuner / Commissioner |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
| 12 | Commence automatic commissioning and place into normal operation | Risk of electric shock | 50 | - All testing and commissioning are conducted by a licenced Electrician. | 10 | Tuner / Commissioner |
|  |  |  |  | - Ensure the area around CB or Controller is clear of trip hazards and conductive material such as water on the floor before de-energisation. |  |  |
|  |  |  |  | - Wear PPE such as riggers gloves, and safety glasses as minimum. Check that the incoming supply is enclosed and switches are insulated (in place if a new Controller is installed). |  |  |
|  |  |  |  | - If not, then use additional PPE including LV gloves and insulated mat. |  |  |
|  |  |  |  | - Have LVR rescue and a second technician available as a spotter if identified as part of risk assessment in line with Electrical COP. |  |  |
|  |  | Risk of fall from height from top of car into shaft/pit | 50 | - Handrails in place on top of the car where the gap between the shaft wall and outside perimeter exceeds 300mm. If none is in place, alternative fall protection is to be used. Harnesses should not be used when the lift car is moving due to the possibility of entanglement. The lift car should be secured and stopped from moving before use. | 10 | Tuner / Commissioner |
|  |  |  |  | - Maintain all body within the limits of the lift car top whilst traveling. |  |  |
|  |  |  |  | - Ensure kickboards and handrails are correctly assembled. |  |  |
|  |  | Risk of crush injury from moving/operating parts. | 40 | - Ensure all guards are in place to protect against moving parts. | 7 | Tuner / Commissioner |
|  |  |  |  | - Wear PPE including eye protection and footwear. |  |  |
|  |  |  |  | - Do not touch moving parts, use hand tools. |  |  |
|  |  |  |  | - Listen for noises and check clearances of moving parts. |  |  |
|  |  | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | Tuner / Commissioner |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
| 13 | Clean down the lift shaft, car, car doors, and landing doors | Risk of manual handling injury to Lift Technician when handling materials and equipment. | 28 | - Use equipment such as trolleys or engineered controls (rigging and hoists) and mechanical lifting devices to move equipment where possible. Technician to have Rigging / Hoisting qualification if this equipment is used. | 14 | Tuner / Commissioner |
|  |  |  |  | - Where this is not possible, use safe lifting principles or two-man lifts. |  |  |
|  |  |  |  | - Ensure work areas and fire exits are clear of material. |  |  |
|  |  | Risk of Laceration or Cuts / Abrasions from handling equipment and tools. | 14 | - Use gloves (rigger type or cut-resistant) when handling equipment. | 7 | Tuner / Commissioner |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of materials/equipment with sharp edges. |  |  |
|  |  | Risk of injury from chemical/substance contact. | 40 | - Use specific PPE (Chemical Gloves) where required. Use oil & lubricants per the Safety Data Sheet | 10 | Tuner / Commissioner |
|  |  |  |  | - Only decant in approved/labelled containers clean up spills with spill kits and dispose of oily rags. |  |  |
|  |  |  |  | - Ensure that adequate ventilation is provided. Avoid generating and inhaling mists. Keep containers closed when not in use. |  |  |
|  |  | Risk of electric shock | 40 | - Lift isolated per TKE SWP 18 | 10 | Tuner / Commissioner |
|  |  |  |  | - Electrical work is undertaken by a Licensed Electrician only. |  |  |
|  |  |  |  | - Guard any circuit that is not de-energized on a locked-out controller. Always isolate the circuit and have the unit locked out before working on the unit |  |  |
|  |  |  |  | - Only approved testing devices and insulated tools are to be used. Inspect tools before use. |  |  |
|  |  | Risk of crush injury from moving/operating parts. | 40 | - Ensure all guards are in place to protect against moving parts. | 7 | Tuner / Commissioner |
|  |  |  |  | - Wear PPE including eye protection and footwear. |  |  |
|  |  |  |  | - Do not touch moving parts, use hand tools. |  |  |
|  |  |  |  | - Listen for noises and check clearances of moving parts. |  |  |
| 14 | General clean-up, and inspection to ensure all items collected and no trip hazards present; and leave the site. | Risk of laceration or cuts/abrasions from handling equipment and tools | 14 | - Relevant PPE to be always worn (safety glasses, steel-capped boots, long-sleeved shirt, long pants) | 7 | TKE Personnel |
|  |  |  |  | - Ensure the work area is clear and free of trip hazards. |  |  |
|  |  |  |  | - Use correct manual handling techniques. |  |  |
|  |  |  |  | - Take note of and care when handling equipment or materials with sharp edges. |  |  |
|  |  | Risk of injury as a result of improper manual handling / awkward position | 28 | - Relevant PPE to be always worn (safety glasses, steel-capped boots, long-sleeved shirt, long pants) | 14 | TKE Personnel |
|  |  |  |  | - Correct Manual handling techniques are used when handling equipment |  |  |
|  |  | Risk of environmental harm or damage | 28 | - Separate waste and recycle relevant materials on site. If not available on-site, return to Store to be disposed of in relevant bins (metals, cardboard, timber, etc) | 14 | TKE Personnel |

| # SWMS Sign Off I have been consulted and allowed to contribute to this SWMS. I confirm that I understood the contents of this SWSM. I have read and understand the relevant SWP related to this SWMS. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | Signature | Date | Name | Signature | Date |
|  |  | / / |  |  | / / |
|  |  | / / |  |  | / / |
|  |  | / / |  |  | / / |
|  |  | / / |  |  | / / |
|  |  | / / |  |  | / / |
|  |  | / / |  |  | / / |
|  |  | / / |  |  | / / |
|  |  | / / |  |  | / / |
|  |  | / / |  |  | / / |
|  |  | / / |  |  | / / |
|  |  | / / |  |  | / / |
|  |  | / / |  |  | / / |
|  |  | / / |  |  | / / |
|  |  | / / |  |  | / / |